Boot time & Memory usage optimization

The main goal of this project is to reduce the total time taken by a Linux-based system to boot from power-on to a usable state (login prompt or GUI).
In simple words:

Make Linux start faster — by identifying delays in the boot sequence and optimizing or removing unnecessary components.

### Why Boot Time Optimization is Needed

Boot time is critical for:

- Embedded Systems (Raspberry Pi, Automotive ECUs, IoT devices)

- Consumer Electronics (Smart TVs, Cameras, Routers)

- Industrial Devices that need quick startup or recovery.

A fast boot improves:

- User experience

- Energy efficiency

- Reliability in time-critical systems

When you power on a Linux system, the boot process involves several stages:

- Bootloader Stage
(e.g., U-Boot, GRUB)

  - Initializes hardware

  - Loads the Linux kernel into memory

  - Passes control to the kernel

  - ➜ Optimization: Reduce bootloader wait time, disable splash/logo delay, optimize device initialization.

- Kernel Initialization Stage

  - Kernel detects and initializes hardware drivers.

  - Mounts the root filesystem (rootfs).

  - Starts init process.

  - ➜ Optimization: Remove unused kernel modules, enable only required drivers, use faster filesystem (like ext4 instead of ext3), or use compressed but fast decompression algorithms (LZ4 instead of gzip).

- User Space Initialization Stage

  - The init system (like systemd, sysvinit, or busybox init) starts essential services.

  - ➜ Optimization: Parallelize startup, disable unused services, use lightweight init systems (like BusyBox).

- Login / GUI Stage

  - System reaches a usable state (shell prompt or GUI).

  - ➜ Optimization: Delay non-essential services until after login.

Steps to kernel optimization  boot time

step 1:

1.  command:       systemd-analyze

above command used to to baseline Boot measurement

Example Output (Before Optimization):

Startup finished in 3.029s (kernel) + 12.301s (userspace) = 15.331s

graphical.target reached after 12.251s in userspace.

Total boot = 15.3 seconds

- Most of the delay (≈80%) is in userspace, i.e. systemd services like network, Bluetooth, GUI, etc.

- Kernel boot time (≈3s) is already optimized enough, so most of your improvement will come from user-space tuning.

Step2:

Identify which services take long

command:

systemd-analyze blame

You’ll get output like:

8.067s NetworkManager-wait-online.service

4.567s wayvnc.service

2.498s e2scrub_reap.service

1.294s udisks2.service

1.214s cups.service

1.105s ModemManager.service

1.063s rpi-eeprom-update.service

928ms NetworkManager.service

832ms dev-mmcblk0p2.device

727ms accounts-daemon.service

686ms systemd-fsck@dev-disk-by\x2dpartuuid-64e81222\x2d01.service

649ms polkit.service

637ms user@1000.service

565ms lightdm.service

546ms avahi-daemon.service

542ms bluetooth.service

532ms plymouth-quit-wait.service

485ms systemd-logind.service

482ms dbus.service

449ms ssh.service

404ms dphys-swapfile.service

369ms rp1-test.service

337ms colord.service

293ms wpa_supplicant.service

289ms glamor-test.service

257ms sshswitch.service

237ms keyboard-setup.service

226ms systemd-udev-trigger.service

185ms systemd-udevd.service

170ms systemd-sysusers.service

140ms systemd-journald.service

107ms systemd-timesyncd.service

105ms systemd-binfmt.service

101ms systemd-random-seed.service

97ms alsa-restore.service

95ms systemd-modules-load.service

94ms systemd-tmpfiles-setup.service

disable the some of the unwanted services :

# 1. Waits for full network connection — causes 8s delay!

sudo systemctl disable NetworkManager-wait-online.service

# 2. Virtual network console — disable unless you’re remote-controlling via VNC

sudo systemctl disable wayvnc.service

# 3. Disk scrub check for ext filesystems — not needed on every boot

sudo systemctl disable e2scrub_reap.service

# 4. USB disk management (only needed for external drives / GUI)

sudo systemctl disable udisks2.service

# 5. Printer service — safe to remove

sudo systemctl disable cups.service

# 6. Modem Manager — used for 3G/4G dongles only

sudo systemctl disable ModemManager.service

# 7. Avahi — network service discovery (not needed for SSH/local)

sudo systemctl disable avahi-daemon.service

# 8. Bluetooth — disable if not using BT devices

sudo systemctl disable bluetooth.service

# 9. Polkit — used for desktop permission dialogs

sudo systemctl disable polkit.service

# 10. Color management for desktop (unneeded)

sudo systemctl disable colord.service

# 11. Plymouth splash screen

sudo systemctl disable plymouth-quit-wait.service

sudo systemctl disable plymouth-start.service

After Disabling:

Reboot and check again:

sudo reboot

Then measure:

command:    systemd-analyze

observe the boot time after boot time:

Startup finished in 2.368s (kernel) + 3.752s (userspace) = 6.120s

graphical.target reached after 3.716s in userspace.

Command:   systemd-analyze blame | head -15

mirafra@mm3:~ $ systemd-analyze blame | head -15

783ms dev-mmcblk0p2.device

762ms accounts-daemon.service

640ms rpi-eeprom-update.service

474ms NetworkManager.service

423ms systemd-fsck@dev-disk-by\x2dpartuuid-64e81222\x2d01.service

396ms udisks2.service

392ms user@1000.service

388ms avahi-daemon.service

341ms dbus.service

337ms systemd-logind.service

330ms lightdm.service

322ms ssh.service

304ms cups.service

299ms plymouth-quit-wait.service

268ms dphys-swapfile.service

To re-enable all you disabled before:

You can just run this list:

sudo systemctl enable NetworkManager-wait-online.service

sudo systemctl enable wayvnc.service

sudo systemctl enable e2scrub_reap.service

sudo systemctl enable udisks2.service

sudo systemctl enable cups.service

sudo systemctl enable ModemManager.service

sudo systemctl enable avahi-daemon.service

sudo systemctl enable bluetooth.service

sudo systemctl enable polkit.service

sudo systemctl enable colord.service

sudo systemctl enable plymouth-quit-wait.service

sudo systemctl enable plymouth-start.service

### Optional — verify all enabled services:

systemctl list-unit-files --state=enabled

## 1. Disable Unused Kernel Modules

Every extra driver or module the kernel loads adds milliseconds or seconds.
To optimize:

sudo nano /etc/modprobe.d/blacklist.conf

Add lines for modules you don’t use, e.g.:

blacklist bluetooth

blacklist snd_bcm2835

blacklist i2c_bcm2835

blacklist spi_bcm2835

Only blacklist modules you’re sure are unused — disabling critical ones can prevent boot.

You can check which modules load:

lsmod

## 2. Remove or Minimize Initramfs

If your kernel uses an initramfs, it adds time to decompress and run initialization scripts.
Raspberry Pi OS often boots without an initramfs — check with:

ls /boot/initrd*

If none found, great — if there is one, and you don’t need it:

- Rebuild kernel to boot directly to rootfs:

- sudo update-initramfs -d -k $(uname -r)

## 3. Optimize cmdline.txt

This file defines boot parameters for the kernel:

sudo nano /boot/cmdline.txt

You can add/remove options to speed things up:

| Parameter | Purpose | Example |
| --- | --- | --- |
| quiet | Reduces console output → slightly faster | quiet |
| fastboot | Skips fsck at boot | fastboot |
| noinitrd | Skip initramfs loading | noinitrd |
| loglevel=3 | Fewer logs → faster console init | loglevel=3 |
| rootwait=0 | Skip long root device wait | only if using fixed root device |

Example optimized line:

//console=serial0,115200 console=tty1 root=PARTUUID=xxxxxxx rootfstype=ext4 //fsck.repair=no //rootwait quiet loglevel=3 noinitrd fastboot

Make it minimal:

console=serial0,115200 console=tty1 root=PARTUUID=<uuid> rootfstype=ext4 fsck.repair=yes rootwait quiet splash loglevel=3

You can remove console=serial0,115200 if not debugging via UART.

Effect: Reduce console output → less time wasted printing logs.

## 4. Disable Splash Screen & Logo

This doesn’t affect much performance but helps visibility:

sudo nano /boot/config.txt

Add:

disable_splash=1

boot_delay=0

check again:

sudo reboot

Then measure:

command:    systemd-analyze

observe the boot time after boot time:

Startup finished in 2.368s (kernel) + 3.752s (userspace) = 6.120s

graphical.target reached after 3.716s in userspace.

memory optimization :

The main goal of this project is to reduce the overall memory consumption of a Linux system and manage memory efficiently, so the system can run faster and handle more processes without running out of RAM.

### Why Memory Optimization is Needed:

Memory is a critical resource in any system. Poor memory management can lead to:

- Slow system performance

- Frequent swapping to disk (high I/O)

- System crashes or kernel panics

- Poor performance in embedded or resource-constrained systems (like Raspberry Pi, IoT devices, routers)

Optimizing memory helps:

- Improve system speed and responsiveness

- Reduce memory fragmentation

- Support more applications or services

- Increase stability for embedded systems

### Brief Description

Linux memory consists of several segments and subsystems:

- Kernel Memory

  - Used by the kernel for its own operations (drivers, data structures, caches).

  - Optimization: Remove unused modules, reduce slab allocations, and use efficient data structures.

- User-Space Memory

  - Memory used by applications.

  - Optimization: Limit memory-hungry processes, improve allocation/deallocation efficiency.

- Caches and Buffers

  - Linux uses free memory for file system caches (page cache, buffer cache).

  - Optimization: Tune vm.swappiness, vm.vfs_cache_pressure, or use tmpfs for temporary data.

- Swap Memory

  - Swap acts as overflow when RAM is full.

  - Optimization: Reduce swap usage, prioritize critical processes in RAM, and adjust swapiness.

chmod +x optimize_memory_full.sh

sudo bash optimize_memory_full.sh

### Optional (View Logs Later)

To view later:

cat /tmp/memory_before.log

cat /tmp/memory_after.log

Save this as: optimize_memory_full.sh

#!/bin/bash

#  Raspberry Pi Full Memory Optimization + Comparison Tool

#  Author: Adi (for Raspberry Pi kernel optimization project)

#  Safe version — no data deletion

LOG_BEFORE="/tmp/memory_before.log"

LOG_AFTER="/tmp/memory_after.log"

# --- Function to collect memory stats ---

collect_stats() {

echo "===== MEMORY & SWAP ====="

free -h

echo

echo "===== zRAM SWAP USAGE ====="

swapon --show

echo

echo "===== KERNEL MEMORY ====="

cat /proc/meminfo | grep -E 'MemTotal|MemFree|Buffers|Cached|Slab'

echo

echo "===== TOP 5 RAM USERS ====="

ps -eo pid,comm,%mem,%cpu --sort=-%mem | head -n 6

echo

echo "===== I/O & CPU USAGE (vmstat 1 3) ====="

vmstat 1 3

echo

}

# --- 1 Collect BEFORE Optimization Stats ---

echo " Collecting BEFORE optimization stats..."

collect_stats > "$LOG_BEFORE"

# --- 2️ Disable Unused Services ---

echo " Disabling unnecessary services..."

SERVICES=(

cups.service

bluetooth.service

avahi-daemon.service

ModemManager.service

NetworkManager-wait-online.service

wayvnc.service

colord.service

plymouth-quit-wait.service

plymouth-start.service

)

for s in "${SERVICES[@]}"; do

sudo systemctl disable $s 2>/dev/null

done

# --- 3️ Tune Swappiness ---

echo " Setting vm.swappiness=10..."

sudo sysctl -w vm.swappiness=10

grep -q "vm.swappiness" /etc/sysctl.conf || echo "vm.swappiness=10" | sudo tee -a /etc/sysctl.conf

# --- 4️ Enable zRAM ---

echo " Checking zRAM installation..."

if ! command -v zramctl &> /dev/null; then

echo "Installing zram-tools..."

sudo apt-get install -y zram-tools

fi

sudo systemctl enable --now zram-config 2>/dev/null || sudo systemctl enable --now zramswap 2>/dev/null

# --- 5️ Blacklist Unused Kernel Modules ---

echo " Blacklisting unused kernel modules..."

sudo bash -c 'cat <<EOF > /etc/modprobe.d/blacklist-memory-optimization.conf

blacklist bluetooth

blacklist snd_bcm2835

# blacklist i2c_bcm2835   # Uncomment only if not using I2C

# blacklist spi_bcm2835   # Uncomment only if not using SPI

EOF'

# --- 6️ Optimize Boot Parameters ---

echo " Optimizing /boot/cmdline.txt..."

sudo cp /boot/cmdline.txt /boot/cmdline.txt.backup

grep -q "fastboot" /boot/cmdline.txt || sudo sed -i 's/$/ fastboot noinitrd loglevel=3 quiet/' /boot/cmdline.txt

# --- 7️ Optional: Disable GUI ---

read -p "Do you want to disable GUI and boot to console? (y/n): " ans

if [[ "$ans" == "y" || "$ans" == "Y" ]]; then

sudo systemctl set-default multi-user.target

echo "GUI disabled — booting to console mode next time."

else

echo "Skipping GUI disable."

fi

# --- 8️ Drop Page Cache ---

echo " Clearing page cache..."

sudo sync

sudo sh -c "echo 3 > /proc/sys/vm/drop_caches"

# --- 9️ Collect AFTER Optimization Stats ---

echo " Collecting AFTER optimization stats..."

collect_stats > "$LOG_AFTER"

# --- 10 Show Comparison ---

echo

echo "==============================================="

echo " BEFORE OPTIMIZATION SUMMARY"

cat "$LOG_BEFORE"

echo "==============================================="

echo "AFTER OPTIMIZATION SUMMARY"

cat "$LOG_AFTER"

echo "==============================================="

echo "Optimization complete! Reboot recommended."

| Tool | Purpose | Related to Boot Time? | Used When |
| --- | --- | --- | --- |
| top | Show CPU/memory usage | No | After boot |
| htop | Interactive process viewer | No | After boot |
| systemd-analyze | Measure total kernel + user boot time | Yes | During boot |
| bootchart | Visualize boot process timeline | Yes | During boot |
| dmesg -T | View kernel init timings | Yes | Kernel boot |
| initcall_debug | Trace kernel function timings | Yes | Kernel boot |
| U-Boot bootdelay/logs | Measure bootloader time | Yes | Bootloader stage |

Tools:

## /proc/meminfo

/proc/meminfo is a virtual file in Linux that shows current memory usage in the system.

You can check it using:

cat /proc/meminfo

It gives detailed information about RAM, swap, buffers, caches, etc.

## rootfs

rootfs = root filesystem, the initial filesystem mounted at boot time.

When Linux boots:

- The kernel mounts an initial rootfs (usually in RAM, tmpfs or initramfs).

- Then it mounts the real root filesystem (like /dev/sda1, /dev/mmcblk0p2, or NFS).

- After the switch, the old rootfs may stay mounted as /run/initramfs or get unmounted.

### Example

On an embedded system (like Raspberry Pi):

- Bootloader loads the kernel and initramfs into RAM.

- initramfs provides basic utilities (/bin/sh, /dev, /sys, /proc).

- Kernel mounts it as rootfs → then mounts the real filesystem from SD card.

Check using:

df -h  or    mount | grep rootfs

## Memory Segments (Different Numbered Sections)

Linux divides memory into different regions depending on process and kernel space

in userspace (per process)

| Segment | Purpose | Example |
| --- | --- | --- |
| Text Segment | Program instructions (read-only) | Compiled code of .text |
| Data Segment | Initialized global/static variables | int x = 5; |
| BSS Segment | Uninitialized global/static variables | int y; |
| Heap | Dynamically allocated memory | malloc(), calloc() |
| Stack | Function calls, local variables | Automatic variables |

In Kernel-Space (shared by all)

| Segment | Description |
| --- | --- |
| Text segment | Kernel code (.text section) |
| Data segment | Kernel data (.data, .bss) |
| Slab cache | Allocator for small kernel objects |
| vmalloc area | Virtual memory for modules/drivers |
| High memory (on 32-bit) | Physical memory not directly mapped into kernel |
| Page cache / Buffers | Used for caching file I/O |
| DMA region | Reserved for Direct Memory Access |

We can check kernel memory info in /proc/meminfo (e.g., Slab, VmallocUsed) and /proc/slabinfo.

### What is rootfs?

rootfs means root filesystem — it’s the first filesystem that Linux uses when it starts running.

When Linux boots, it needs a place where all files like /bin, /dev, /etc, /home, etc. live.
That place is called the root filesystem — written as /.

### How it works during boot

- Kernel starts first.

  - The kernel itself doesn’t have access to files yet.

  - It only knows how to talk to hardware.

- Kernel mounts a temporary root filesystem (rootfs).

  - Usually stored in RAM (called initramfs).

  - Contains small tools needed to start the real system.

  - Example: /init, /bin/sh, /dev, /proc.

- Kernel then mounts the real root filesystem.

  - From disk, SD card, or NFS network.

  - Example: /dev/sda2, /dev/mmcblk0p2, or network location.

- Switches to that real root filesystem.

  - Then your Linux system starts (systemd, login, etc).