I2C

What is I²C? Explain its working principle.

I²C (Inter-Integrated Circuit) is a synchronous, multi-master, multi-slave, serial communication protocol developed by Philips (now NXP Semiconductors). It is mainly used for communication between microcontrollers and peripheral devices like sensors, EEPROMs, RTCs, etc., over short distances.

Features of I²C:

- Two-wire interface:

  - SDA (Serial Data Line) – for data transmission.

  - SCL (Serial Clock Line) – for clock signal generated by the master.

- Supports multiple masters and slave

- Synchronous – clock is shared (In the case of I²C, the master device generates the clock signal on the SCL (Serial Clock Line). Because both the master and slave devices use this same clock to know when to send or receive bits, they are said to be "synchronized.")

- Address-based communication: Each slave has a unique 7-bit or 10-bit address.

- Speed modes:

  - Standard: 100 kbps

  - Fast: 400 kbps

  - Fast Plus: 1 Mbps

  - High-Speed: 3.4 Mbps

- Because of 1data line it supports half duplex communication

- It is defacto standard.

- Msb bit transferred first.

standard I²C Frame Format

An I²C communication frame consists of several key parts transmitted sequentially over the bus:

1. Start Condition (S)

- Indicates the beginning of a data transfer.

- Generated by the master.

- SDA line transitions from HIGH to LOW while SCL is HIGH.

2. Address Frame

- The master sends a 7-bit (or 10-bit) address of the target slave device.

- Followed by 1 bit called R/W bit:

  - 0 = Write operation

  - 1 = Read operation

3. ACK/NACK Bit

- The slave device whose address matches sends an ACK (Acknowledge) bit by pulling SDA LOW during the 9th clock pulse.

- If no device acknowledges, the SDA stays HIGH (NACK).

4. Data Frame(s)

- Data is transferred byte by byte (8 bits each).

- Each byte is followed by an ACK/NACK bit sent by the receiver (could be slave or master depending on direction).

5. Stop Condition (P)

- Indicates the end of communication.

- Generated by the master.

- SDA line transitions from LOW to HIGH while SCL is HIGH.

Summary Table of the Frame Format:

| Part | Description | Number of Bits |
| --- | --- | --- |
| Start Condition (S) | Signals start of communication | — |
| Address + R/W bit | 7-bit slave address + 1-bit read/write indicator | 8 bits |
| ACK/NACK bit | Acknowledge or Not Acknowledge from slave | 1 bit |
| Data Byte | Data payload sent byte-wise | 8 bits per byte |
| ACK/NACK bit | Acknowledge or Not Acknowledge from receiver | 1 bit per byte |
| Stop Condition (P) | Signals end of communication | — |

What are the lines used in I²C communication

I²C communication uses two bi-directional open-drain lines:

- SDA (Serial Data Line):

  - Carries the data bits being transmitted between devices.

  - It is a bidirectional line, meaning both master and slave can send or receive data.

- SCL (Serial Clock Line):

  - Carries the clock signal generated by the master to synchronize data transfer.

  - All devices use this clock to time when to read or write bits on the SDA line.

- Both lines are open-drain/open-collector, which means devices can only pull the line LOW; the lines are pulled HIGH by pull-up resistors externally.

  - This setup allows multiple devices to share the same bus safely without electrical conflicts.

What do you mean by master and slave in I²C?

Master:

- The device that initiates and controls the communication.

- It generates the clock signal (SCL).

- It starts and stops the data transfer by sending start and stop conditions.

- It addresses the slave devices to communicate with them.

- Can send or receive data depending on the operation.

Slave:

- The device that responds to the master’s requests.

- It listens to the bus and waits for its address.

- It receives commands from the master and sends or receives data accordingly.

- Multiple slaves can be connected on the same I²C bus, each having a unique address.

How is data transferred in I²C?

Frame format explanation

What is the difference between I²C, SPI, and UART?

What is an I²C address? How many devices can be connected?

What is an I²C Address?

- An I²C address is a unique identifier assigned to each slave device on the I²C bus.

- It allows the master device to specify which slave it wants to communicate with during data transfer.

- The address is sent right after the start condition by the master.

- I²C commonly uses 7-bit addressing, where 7 bits represent the address, followed by 1 bit for read/write.

- Some devices support 10-bit addressing for a larger address space, but 7-bit is most common.

How Many Devices Can Be Connected on an I²C Bus?

- Using 7-bit addressing, up to 2⁷ = 128 addresses are possible (0 to 127).

- However, some addresses are reserved for special purposes, so the practical number of devices is slightly less (usually around 112–120 devices).

- With 10-bit addressing, many more devices (over 1000) can be addressed.

- Also, multiple devices can share the same bus lines because the bus is multiplexed by addresses.

Explain the role of start and stop conditions in I²C.

1. Start Condition (S)

- The start condition signals the beginning of a communication frame on the I²C bus.

- It is generated by the master device.

- It occurs when the SDA line transitions from HIGH to LOW while SCL is HIGH.

- This alerts all connected slave devices that a communication is about to begin and that they should listen for the address.

- It ensures that the bus is ready and controlled by the master before data transfer starts.

2. Stop Condition (P)

- The stop condition signals the end of communication on the I²C bus.

- It is also generated by the master device.

- It occurs when the SDA line transitions from LOW to HIGH while SCL is HIGH.

- This releases the bus, indicating that the communication frame is complete.

- After the stop condition, the bus is free for other masters or devices to use.

What is the purpose of ACK and NACK in I²C?

Purpose of ACK and NACK in I²C

In I²C communication, after each 8-bit byte of data is transferred, the receiver sends back an extra bit called:

- ACK (Acknowledge)

- NACK (Not Acknowledge)

These bits are used for handshaking between sender and receiver to ensure reliable data communication.

🔹 ACK (Acknowledge)

- Value: Logic LOW (0)

- Meaning:
The receiver successfully received the byte and is ready for the next one.

- When used:
After each byte of address or data is sent.

- Who sends it?

  - If master sends data → slave sends ACK.

  - If slave sends data → master sends ACK.

🔹 NACK (Not Acknowledge)

- Value: Logic HIGH (1)

- Meaning:
The receiver did not receive the byte properly, or it wants to end the data transfer.

- When used:

  - Typically sent after the last byte to indicate no further data is expected.

  - Also used if the slave device doesn’t recognize the addres

What is the maximum speed of I²C communication?

What are the advantages and disadvantages of using I²C?

Advantages of I²C:

- Simple Two-Wire Interface
→ Only SDA and SCL lines are needed, reducing pin count and complexity.

- Supports Multiple Devices
→ Many slave devices can be connected using unique addresses.

- Master-Slave Communication
→ Master controls communication; avoids bus conflicts.

- Built-in Acknowledge Mechanism
→ Reliable data transfer with ACK/NACK feedback.

- Flexible Data Transfer
→ Supports both read and write operations.

- Multi-Master Capable
→ More than one master can control the bus (with arbitration).

⚠️ Disadvantages of I²C:

- Slower than SPI
→ Max speed is lower compared to SPI (which can go beyond 10+ Mbps).

- Limited Distance
→ Effective over short distances due to capacitance and signal degradation.

- Open-Drain Pull-Up Requirement
→ Requires external pull-up resistors, which affect speed and power.

- More Protocol Overhead
→ Start/Stop conditions, addressing, and ACK/NACK add overhead.

- Bus Contention in Multi-Master
→ Complex logic needed for arbitration and clock stretching.

- How do clock stretching and arbitration work in I²C?

Clock Stretching

Purpose:
Clock stretching allows a slave device to slow down the master if it needs more time to process data.

How it works:

- The master generates the clock (SCL).

- After a clock pulse, if the slave needs more time, it holds the SCL line LOW.

- The master waits until the slave releases the SCL line (goes HIGH).

- Only then can the master proceed to send or receive the next bit.

When it's used:

- When the slave is a slower device (e.g., EEPROM or sensor).

- To allow internal processing time or data fetching.

Important:

Why is Clock Stretching Needed?

- Slave Devices May Be Slower:

  - Not all slave devices operate at the same speed as the master.

  - Example: A temperature sensor might need time to convert analog data to digital.

- Avoid Data Loss:

  - Without clock stretching, if a slave is not ready when data arrives, it may miss bits, leading to data corruption.

- Synchronous Communication:

  - I²C is synchronous — data changes on SDA are controlled by the clock (SCL).

  - If the slave isn't ready, it must pause the clock to prevent the master from sending the next bit too soon.

- Real-Time Data Handling:

  - Some devices fetch data from internal memory or perform calculations before responding — they need time to do that.

SCL is an open-drain line with a pull-up resistor. Any device can pull it low to stretch the clock.

What is Arbitration in I²C?

Arbitration is a mechanism used in multi-master I²C systems to ensure that only one master controls the bus at a time, even if multiple masters try to start communication simultaneously.

🔹 Why Arbitration Is Needed:

- I²C supports multiple masters on the same bus.

- If two masters try to communicate at the same time, arbitration prevents bus conflict and data corruption.

🔹 How Arbitration Works (Bit-by-Bit):

- Two (or more) masters generate a START condition at the same time.

- Each master starts transmitting the address byte (and possibly data).

- SDA is open-drain:

  - If one master sends 1 and another sends 0, the line stays at 0 (LOW wins).

- Every master monitors SDA while transmitting:

  - If a master tries to send 1 but sees a 0, it knows another master is transmitting a more dominant bit.

  - That master immediately stops transmitting and loses arbitration.

- The master that successfully transmits all bits without seeing a mismatch wins and continues communication.

🔹 Important Points:

| Feature | Description |
| --- | --- |
| Bit-by-bit check | Arbitration is checked at every bit. |
| Dominant bit | LOW (0) is dominant over HIGH (1). |
| SDA line monitored | Each master reads SDA while writing. |
| Non-destructive | No data collision occurs; only one wins. |

- What happens if two I²C masters try to communicate at the same time?

- How does the master identify a specific slave device on the I²C bus?

In I²C communication, each slave device on the bus has a unique 7-bit or 10-bit address. The master identifies and communicates with a specific slave by sending a START condition followed by the slave address and a Read/Write bit. Only the slave with the matching address sends an acknowledgment (ACK) and engages in communication. Other devices ignore the transmission. This addressing mechanism ensures proper device selection even when multiple devices share the same bus lines."

- What is the significance of 7-bit vs 10-bit addressing in I²C?

- Explain the I²C read and write operation sequence.

- Can multiple masters be used in I²C? How is bus control managed?

I²C supports multiple masters on the same bus, but only one master can control the bus at a time.Bus control is managed using a process called arbitration. If two masters attempt to communicate simultaneously, they monitor the SDA line while transmitting. If a master detects a mismatch—like sending a logic '1' but reading a '0'—it immediately stops transmitting, allowing the other master to continue.This arbitration is non-destructive, meaning no data is corrupted during the process. Additionally, clock synchronization ensures that the communication remains stable even when different masters have slightly different clock rates.

- What is the role of pull-up resistors in I²C? Why are they needed?

"Pull-up resistors play a crucial role in I²C communication. Both SDA and SCL lines are open-drain, meaning the devices on the bus can only pull the lines LOW; they cannot drive them HIGH.Because of this, external pull-up resistors are required to pull the lines to a logic HIGH level when no device is actively pulling them LOW.Without these resistors, the lines would float and could result in unreliable logic level detection, which may cause communication errors or failure.

- What is bus arbitration in I²C? How is it resolved?

- How do you handle an I²C device that is not responding?

- What are repeated start conditions in I²C? Why are they used?

DATA VALIDITY & TRANSITION

Simple Rule for Data Validity in I²C

- SCL = HIGH → SDA must be stable (no change) → data is valid.

- SCL = LOW → SDA can change → data is allowed to change.

⚡ 2. Special Transitions (START / STOP)

| Condition | When it happens | What it means |
| --- | --- | --- |
| START | SDA goes HIGH → LOW while SCL is HIGH | Start communication |
| STOP | SDA goes LOW → HIGH while SCL is HIGH | Stop communication |

🧮 3. When a Bit is Detected

- Each bit is read by the receiver when SCL is HIGH.

- The sender places the bit on SDA when SCL is LOW (setup time).

- The receiver samples (reads) the bit when SCL goes HIGH.

SCL = LOW → Master/Slave sets SDA (data)

SCL ↑ (rising edge) → Receiver reads SDA (bit is detected)

4. ACK Bit Detection

- After sending 8 data bits, the receiver must send 1 ACK bit.

- ACK is detected as:

  - SCL = HIGH, and

  - SDA = LOW → means ACK (acknowledge received)

  - SDA = HIGH → means NACK (no acknowledge)

ACK Bit = 9th clock pulse

| Bit No. | Purpose |
| --- | --- |
| 1–8 | Data bits |
| 9 | ACK/NACK bit |