## 1. copy_from_user()

### 📌 What it is:

- copy_from_user() is a kernel function used to safely copy data from user space to kernel space.

- It prevents direct access to user-space pointers inside kernel code.

### 🔧 Prototype:

long copy_from_user(void *kernel_buffer, const void __user *user_buffer, unsigned long count);

to: Kernel-space buffer to store the data.

- from: User-space pointer.

- n: Number of bytes to copy.

"copy_from_user() is used to transfer data from user space to kernel space. It returns non-zero on failure (e.g., if the pointer is invalid)."

// Write

static ssize_t my_write(struct file *file, const char __user *user_buf, size_t len, loff_t *offset) {

if (len >= BUF_SIZE)

len = BUF_SIZE - 1;

if (copy_from_user(kernel_buffer, user_buf, len))

return -EFAULT;

kernel_buffer[len] = '\0';  // Null-terminate

printk(KERN_INFO "Received from user: %s\n", kernel_buffer);

return len;

}

## 2. copy_to_user()

### 📌 What it is:

- copy_to_user() is used to send data from kernel space back to user space safely.

- Ensures memory safety and prevents kernel from directly writing to user-space addresses.

### 🔧 Prototype:

### long copy_to_user(void __user *user_buffer, const void *kernel_buffer, unsigned long count);

to: User-space pointer to store result.

- from: Kernel buffer.

- n: Number of bytes to copy.

copy_to_user() is used to send data from the kernel to a user-space variable. It ensures the destination is valid and avoids segmentation faults."

/ /Read

static ssize_t my_read(struct file *file, char __user *user_buf, size_t len, loff_t *offset) {

size_t data_len = strlen(kernel_buffer);

if (copy_to_user(user_buf, kernel_buffer, data_len))

return -EFAULT;

return data_len;

}

--------------------------------------------------------------------------------------------------------------------

### What is ioctl()?

ioctl stands for I/O control, and it's a system call used in device drivers to send control commands between user space and kernel space — beyond just read() and write() operations.

static long my_ioctl(struct file *file, unsigned int cmd, unsigned long arg);

struct file *file – Pointer to the file structure representing the open device file.

cmd – the command number defined by _IO, _IOW, _IOR, or _IOWR.

arg – typically a pointer to user-space data to pass or receive.

Usage of ioctl():

#define IOCTL_SET_NUM _IOW(MAJOR_NUM, 0, int)

#define IOCTL_GET_NUM _IOR(MAJOR_NUM, 1, int)

static int value = 0;

static long my_ioctl(struct file *file, unsigned int cmd, unsigned long arg) {

switch (cmd) {

case IOCTL_SET:

if (copy_from_user(&value, (int __user *)arg, sizeof(int)))

return -EFAULT;

printk(KERN_INFO "IOCTL SET: %d\n", value);

break;

case IOCTL_GET:

if (copy_to_user((int __user *)arg, &value, sizeof(int)))

return -EFAULT;

printk(KERN_INFO "IOCTL GET: %d\n", value);

break;

default:

return -EINVAL;

}

return 0;

}

------------------------------------------------------------------------------------------------------------------------

## ✅ What is a Tasklet?

A tasklet is a bottom-half mechanism in the Linux kernel used to defer work that should not be done in interrupt context.
Tasklets run after an interrupt handler, in softirq context, and are used for non-time-critical tasks.

## ✅ Why use Tasklets?

- Interrupt handlers must finish quickly → don't do heavy work there.

- Tasklets are scheduled from interrupt context, but executed later.

- They allow deferring processing while staying in kernel space.

Tasklet is a bottom-half mechanism used to defer work from interrupt context.

- Defined using DECLARE_TASKLET(name, func, data).

- Scheduled using tasklet_schedule(&tasklet).

- Canceled using tasklet_kill(&tasklet) in cleanup.

ISR (Interrupt Service Routine) will not do heavy work — it just schedules a tasklet.

The tasklet function handles the deferred processing.

#include <linux/module.h>

#include <linux/kernel.h>

#include <linux/interrupt.h>

#define IRQ_NO 1  // Typically keyboard IRQ (used for testing only)

// Tasklet function prototype

void my_tasklet_func(unsigned long data);

// Declare and initialize tasklet

DECLARE_TASKLET(my_tasklet, my_tasklet_func, 0);

// Tasklet Function (Bottom Half)

void my_tasklet_func(unsigned long data) {

pr_info("Tasklet executed: deferred work after interrupt\n");

}

// Interrupt Handler (Top Half)

irqreturn_t my_irq_handler(int irq, void *dev_id) {

pr_info("Interrupt Occurred: IRQ %d\n", irq);

// Defer work to tasklet

tasklet_schedule(&my_tasklet);

return IRQ_HANDLED;

}

// Module Init

static int __init my_tasklet_irq_init(void) {

int ret;

pr_info("Tasklet + IRQ module loading...\n");

// Request IRQ (shared = true)

ret = request_irq(IRQ_NO, my_irq_handler, IRQF_SHARED, "my_irq_tasklet", (void *)(my_irq_handler));

if (ret) {

pr_err("Cannot register IRQ %d\n", IRQ_NO);

return ret;

}

pr_info("IRQ %d registered successfully\n", IRQ_NO);

return 0;

}

// Module Exit

static void __exit my_tasklet_irq_exit(void) {

// Free IRQ and kill tasklet

free_irq(IRQ_NO, (void *)(my_irq_handler));

tasklet_kill(&my_tasklet);

pr_info("Tasklet + IRQ module unloaded\n");

}

module_init(my_tasklet_irq_init);

module_exit(my_tasklet_irq_exit);

MODULE_LICENSE("GPL");

MODULE_AUTHOR("Swarna");

MODULE_DESCRIPTION("Tasklet example triggered by IRQ");

------------------------------------------------------------------------------------------------------------------------

## What is a Workqueue?

A workqueue is a mechanism in the Linux kernel used to schedule work to be executed in process context, instead of interrupt context.

### 🔧 Why Use Workqueues?

- Unlike tasklets, workqueues can sleep, block, or access user-mode memory.

- Suitable for tasks that may take longer or need to use functions that may sleep (e.g., file operations, memory allocation).

## ✅ Key Differences: Tasklet vs Workqueue

| Feature | Tasklet | Workqueue |
| --- | --- | --- |
| Context | Softirq (atomic) | Process context |
| Can Sleep | ❌ No | ✅ Yes |
| Blocking Ops | ❌ Not Allowed | ✅ Allowed |
| Execution | ASAP after interrupt | Can be delayed |
| Use Case | Quick deferral | Complex, slow, or blocking tasks |
| API | tasklet_schedule() | schedule_work(), queue_work() |

#include <linux/module.h>

#include <linux/kernel.h>

#include <linux/interrupt.h>

#include <linux/workqueue.h>

#define IRQ_NO 1  // Example IRQ (keyboard)

static struct work_struct my_work;

// Workqueue handler (bottom half)

void workqueue_func(struct work_struct *work)

{

pr_info("Workqueue: deferred work executed in process context\n");

}

// IRQ handler (top half)

irqreturn_t my_irq_handler(int irq, void *dev_id)

{

pr_info("IRQ %d triggered. Scheduling workqueue.\n", irq);

// Schedule bottom half using workqueue

schedule_work(&my_work);

return IRQ_HANDLED;

}

// Module init

static int __init workqueue_example_init(void)

{

int ret;

pr_info("Loading workqueue + IRQ module...\n");

INIT_WORK(&my_work, workqueue_func);

ret = request_irq(IRQ_NO, my_irq_handler, IRQF_SHARED, "my_workqueue_irq", (void *)(my_irq_handler));

if (ret) {

pr_err("Failed to register IRQ %d\n", IRQ_NO);

return ret;

}

pr_info("IRQ %d registered and workqueue ready\n", IRQ_NO);

return 0;

}

// Module exit

static void __exit workqueue_example_exit(void)

{

free_irq(IRQ_NO, (void *)(my_irq_handler));

cancel_work_sync(&my_work);  // Waits for work to finish or cancel it

pr_info("Workqueue + IRQ module unloaded\n");

}

module_init(workqueue_example_init);

module_exit(workqueue_example_exit);

MODULE_LICENSE("GPL");

MODULE_AUTHOR("Swarna");

MODULE_DESCRIPTION("Simple Workqueue example triggered by interrupt");

A workqueue runs in process context, allowing it to sleep or block.

- Initialized using INIT_WORK(&wq, func);

- Scheduled using schedule_work(&wq);

- Use cancel_work_sync() in exit to clean up.

------------------------------------------------------------------------------------------------------------------------

## What is a Wait Queue?

A wait queue is a kernel mechanism used to put a process to sleep until a specific condition becomes true, usually signaled by an interrupt or another kernel thread.

## ✅ Why Use Wait Queues?

- To block a process (e.g., read() or ioctl() call) until data is ready.

- Prevent busy waiting (CPU spinning).

- Often used with hardware events, shared buffers, or producer-consumer logic.

## Important APIs

DECLARE_WAIT_QUEUE_HEAD(wq);               // Declare a wait queue

wait_event(wq, condition);                 // Sleep until condition is true

wait_event_interruptible(wq, condition);   // Same but signal-safe

wake_up(&wq);                              // Wake up one or more sleepers

#include <linux/module.h>

#include <linux/kernel.h>

#include <linux/init.h>

#include <linux/wait.h>

#include <linux/sched.h>

#include <linux/kthread.h>

#include <linux/delay.h>

static DECLARE_WAIT_QUEUE_HEAD(my_queue);

static int condition = 0;

static struct task_struct *my_thread;

// Kernel thread that waits

int wait_thread_fn(void *data)

{

pr_info("Thread: Waiting for condition...\n");

// Sleep until 'condition' becomes non-zero

wait_event_interruptible(my_queue, condition != 0);

pr_info("Thread: Woken up! Condition = %d\n", condition);

return 0;

}

// Module init

static int __init waitqueue_example_init(void)

{

pr_info("Wait Queue module loaded\n");

// Start thread

my_thread = kthread_run(wait_thread_fn, NULL, "wait_thread");

// Simulate delay and then wake up

msleep(3000);

condition = 1;

wake_up(&my_queue);

return 0;

}

// Module exit

static void __exit waitqueue_example_exit(void)

{

pr_info("Wait Queue module unloaded\n");

}

module_init(waitqueue_example_init);

module_exit(waitqueue_example_exit);

MODULE_LICENSE("GPL");

MODULE_AUTHOR("Swarna");

MODULE_DESCRIPTION("Simple wait queue example");

------------------------------------------------------------------------------------------------------------------------

SIMPLE CHARACTER DRIVER:

#include <linux/init.h>

#include <linux/module.h>

#include <linux/fs.h>

#include <linux/uaccess.h>

#define DEVICE_NAME "mychar"

#define BUF_SIZE 1024

MODULE_LICENSE("GPL");

MODULE_AUTHOR("swarna");

MODULE_DESCRIPTION("Simple Character Driver");

static int major;

static char kernel_buffer[BUF_SIZE];

static int open_count = 0;

// Open

static int my_open(struct inode *inode, struct file *file) {

open_count++;

printk(KERN_INFO "Device opened %d time(s)\n", open_count);

return 0;

}

// Read

static ssize_t my_read(struct file *file, char __user *user_buf, size_t len, loff_t *offset) {

size_t data_len = strlen(kernel_buffer);

if (copy_to_user(user_buf, kernel_buffer, data_len))

return -EFAULT;

return data_len;

}

// Write

static ssize_t my_write(struct file *file, const char __user *user_buf, size_t len, loff_t *offset) {

if (len >= BUF_SIZE)

len = BUF_SIZE - 1;

if (copy_from_user(kernel_buffer, user_buf, len))

return -EFAULT;

kernel_buffer[len] = '\0';  // Null-terminate

printk(KERN_INFO "Received from user: %s\n", kernel_buffer);

return len;

}

// Close

static int my_release(struct inode *inode, struct file *file) {

printk(KERN_INFO "Device closed\n");

return 0;

}

// File operations structure

static struct file_operations fops = {

.owner = THIS_MODULE,

.open = my_open,

.read = my_read,

.write = my_write,

.release = my_release,

};

// Module init

static int __init mychar_init(void) {

major = register_chrdev(0, DEVICE_NAME, &fops);

if (major < 0) {

printk(KERN_ALERT "Failed to register device\n");

return major;

}

printk(KERN_INFO "Registered device '%s' with major number %d\n", DEVICE_NAME, major);

return 0;

}

// Module exit

static void __exit mychar_exit(void) {

unregister_chrdev(major, DEVICE_NAME);

printk(KERN_INFO "Unregistered device '%s'\n", DEVICE_NAME);

}

module_init(mychar_init);

module_exit(mychar_exit);

------------------------------------------------------------------------------------------------------------------------

### ✅ Upper Half (Top Half) :

- The upper half is the interrupt service routine (ISR) in Linux.

- It is executed immediately when an interrupt occurs.

- Runs in interrupt context, so it must be very fast and non-blocking.

- Cannot call functions that sleep or perform long tasks.

- Typically used to acknowledge the interrupt and schedule the bottom half.

### ✅ Bottom Half :

- The bottom half handles the deferred work after the ISR.

- It runs in softirq or process context, depending on the mechanism.

- Common types: Tasklet, Workqueue, and Softirq.

- Suitable for time-consuming or blocking operations.

- It is scheduled by the upper half using tasklet_schedule() or schedule_work().