A device driver is a special program in the operating system that acts as a bridge between hardware and software. It allows the OS and user applications to communicate with hardware devices (like keyboard, printer, or sensor) without needing to know the low-level hardware details. Drivers handle sending commands to the device, receiving data back, and managing errors. Without device drivers, the OS cannot properly control hardware.

👉 Example: A printer driver translates the "Print" command from your software into the specific signals the printer understands.

A character device driver is a type of device driver that allows data to be transferred one character (byte) at a time between the user application and the hardware device.
It provides an interface to devices like serial ports, keyboards, or sensors, where data is read/written sequentially.
Unlike block device drivers, character drivers do not deal with large chunks of data or random access; instead, they work on a stream-based model.
In Linux, these devices are usually represented as character device files under /dev (for example, /dev/tty, /dev/null).
They interact with the kernel using important functions like open(), read(), write(), ioctl(), and release().
Character drivers are widely used in embedded systems because many peripherals like UART, I2C devices, and sensors communicate in a character-by-character manner.
Thus, a character device driver essentially acts as a bridge between hardware and user space, ensuring smooth byte-wise data communication.

A block device driver is a driver that manages devices which store or transfer data in fixed-size blocks (sectors), rather than one byte at a time.
Typical examples include hard disks, SSDs, USB drives, and SD cards, where data can be accessed randomly in chunks.
Unlike character drivers, block drivers support buffering and caching to improve performance during read/write operations.
They allow the operating system to access data using block numbers instead of sequential streams, enabling random access.
In Linux, block devices are represented under /dev (e.g., /dev/sda), and they play a crucial role in filesystems and persistent storage.

1. Character Driver (No buffering, no caching)

- Works one byte (character) at a time.

- Each read/write call directly transfers data between user program ↔ hardware.

- Example: serial port, keyboard → if you type A, the driver immediately sends that single character to the application.

- No waiting, no storing, no optimization.

2. Block Driver (Buffering + Caching used)

- Works with blocks of data (like 512 bytes, 4 KB, etc.).

- Uses buffering → temporarily stores data in memory before sending to hardware.

  - Example: instead of writing 1 byte at a time to a hard disk (slow), the OS collects 4 KB of data in a buffer and writes it in one go (fast).

- Uses caching → keeps recently read data in RAM so that if the same data is requested again, it can be served quickly without accessing the slow disk.

  - Example: when you open a file, its contents are cached in RAM, so reopening it is faster.

🔑 Meaning of that sentence:

“Unlike character drivers, block drivers support buffering and caching to improve performance” →

- Block drivers optimize speed by grouping reads/writes (buffering) and reusing data already read (caching).

- Character drivers, on the other hand, just send/receive data immediately without optimization.

👉 Think:

- Character driver = instant messaging (send each letter immediately).

- Block driver = parcel delivery (collect items in a box, then send in bulk, and keep a copy at home for reuse).

User Space

- It is the memory area where applications (like browsers, text editors, user programs) run.

- Programs in user space cannot directly access hardware or kernel memory; they must use system calls (like read(), write(), open()) to request services from the kernel.

- Crashes in user space affect only that application, not the whole system.

- It has restricted privileges for safety and stability.

- Examples: /bin/ls, cat file.txt, Python/C programs, etc.

🔹 Kernel Space

- It is the privileged memory area where the operating system core (kernel, drivers, interrupt handlers) runs.

- Kernel space can directly access hardware, memory, and CPU instructions.

- Crashes in kernel space (like a buggy driver) can cause the entire system to hang or crash (kernel panic).

- It has full privileges (Ring 0 in x86 architecture).

- Examples: Linux kernel, device drivers, interrupt service routines, file system handlers.

User space → This is the protected region where normal applications execute with limited privileges, ensuring they cannot directly harm the system or access hardware. Programs here must communicate with the kernel through system calls for any hardware or low-level operations.

Kernel space → This is the highly privileged region where the operating system core and device drivers run, having complete control over hardware and system resources. Any errors here can crash the entire system since it operates with unrestricted access.

⚡ Key Difference in One Line:

👉 User space is for applications with limited privileges, while kernel space is for the OS core with full hardware access and high privileges.

register_chrdev vs alloc_chrdev_region:

Both are used to allocate major and minor numbers for character devices.

register_chrdev():

int register_chrdev(unsigned int major, const char *name, const struct file_operations *fops);

- Every device in Linux is identified by major:minor pair.

- Major number → tells the kernel which driver handles the device.

- Minor number → used inside the driver to differentiate between multiple devices handled by the same driver.

If you pass major = 0
→ kernel will allocate a dynamic unused major number.
→ Example: It may assign major 240 (dynamic majors are usually in 240–511 range).
→ This is printed in dmesg and listed in /proc/devices.

If you pass major = 60 (non-zero)
→ kernel tries to give you exactly major 60.
→ If already taken by another driver, your call fails with -EBUSY.
→ If free, you get major 60.

⚠️ Note: With register_chrdev() you only register major, not specific minors.
The kernel implicitly assumes you handle minors 0–255 for that major.
You don’t have control to request “only minor 0–3” → you must handle the full range.

2. Name

- String you pass ("mychardev") is registered under /proc/devices.

- This is how lsmod/cat /proc/devices shows your driver’s entry.

Example:

$ cat /proc/devices

Character devices:

1 mem

60 mychardev

So mychardev is the label tied to that major number.

3. File Operations Table (fops)

- This is a struct file_operations pointer you give.

- Contains callbacks → open, read, write, ioctl, etc.

- This table is registered at the same time as the major.

That means:

- When a process opens a device file /dev/mydev with your major,

- The kernel looks up → “which fops is tied to this major?”

- And then calls your registered fops->open, fops->read, etc.

4. ✅ One-step vs ❌ Limitations

✔️ One-step:

- register_chrdev() ties major number and fops table in a single call.

- So you don’t need cdev_init() and cdev_add().

❌ Limitation:

- You cannot say “I only want minor 0–3” → kernel assumes you own 0–255.

- If you want multiple devices with separate minor ranges → very messy.

- That’s why modern drivers use alloc_chrdev_region() + cdev_add() (flexible).

What happens with register_chrdev()

- When you register a major with register_chrdev(), the kernel says:

“Okay, here’s your major. You now own all minors 0–255 under this major.”

- That means:

  - If a user creates /dev/mydev0, /dev/mydev1, … /dev/mydev255,

  - The kernel will route all of them to your driver.

  - Even if your driver really only supports 1 or 2 devices.

So the kernel doesn’t restrict the minors for you — you must reject unused ones manually inside your driver.

🔹 2. Why is this a limitation?

- Wasted namespace → You’re forced to occupy 256 minors, even if you need only a few (say, 4).

- Extra code in driver → You must check the minor in every open() and reject invalid ones:

int minor = iminor(inode);

if (minor >= 4)

return -ENODEV;   // Reject

register_chrdev() forces your driver to cover the full minor range (0–255), even if you only need a few; so you must manually check iminor(inode) and reject unwanted minors.

alloc_chrdev_region()

int alloc_chrdev_region(dev_t *dev, unsigned baseminor,

unsigned count, const char *name);

- dev → kernel writes back the allocated device number (major + first minor).

- baseminor → starting minor number (e.g., 0).

- count → how many consecutive minor numbers you want.

- name → name in /proc/devices.

✅ Gives you full control of major + minor numbers.
✅ Preferred modern way with cdev mechanism.
❌ You must separately register file operations using cdev_init() + cdev_add().

| Feature | register_chrdev() | alloc_chrdev_region() |
| --- | --- | --- |
| API style | Old, simple | Modern, flexible |
| Major allocation | Fixed (unless you pass 0) | Always dynamic (unless you combine with register_chrdev_region) |
| Minor numbers | Implicit 0–255 (no choice) | You specify base minor + count |
| Registers fops? | Yes, directly | No → must use cdev_init + cdev_add |
| Use case | Small/simple drivers | Standard for all new drivers |
| Flexibility | Low | High |

register_chrdev()
register_chrdev() is the older way of registering a character driver, where you either specify a fixed major number or pass 0 to let the kernel assign one dynamically.
The drawback is that it implicitly reserves all 256 minors (0–255) for that major, even if the driver only needs a few, which wastes the minor space.
Because of this, the driver must manually check iminor(inode) and reject any minors it doesn’t support.
This makes the driver design less flexible and harder to scale when multiple devices are involved.
Hence, register_chrdev() is simple but considered limited and outdated for modern Linux drivers.

alloc_chrdev_region()
alloc_chrdev_region() is the modern and recommended API that dynamically allocates a major number from the kernel’s 240–511 range.
Unlike register_chrdev(), it allows the driver to specify not just the major but also the exact range of minors (for example, 0–3).
This avoids wasting the full 256 minor numbers and gives precise control over how many device instances are supported.
It makes the driver cleaner, more modular, and better aligned with multiple device scenarios.
That’s why alloc_chrdev_region() is preferred in production-quality drivers.

| Feature | register_chrdev() | alloc_chrdev_region() |
| --- | --- | --- |
| API Age | Older, legacy method | Modern, recommended |
| Major Number | Fixed major (or dynamic if 0, but less flexible) | Dynamically allocated major (from 240–511) |
| Minor Numbers | Reserves all 256 minors (0–255) by default | Can request an exact range of minors (e.g., 0–3) |
| Flexibility | Wastes minor space; must reject unused minors manually | Efficient, allocates only what is needed |
| Usage | Simple for single devices, but limited for multiple | Scalable, clean, and preferred for production drivers |

When we say “all minors wasted” in register_chrdev(), it does not mean physical memory is wasted. It means:

- The kernel reserves the entire minor number range (0–255) for that major device.

  - Example: If you call register_chrdev(250, "mydev", &fops); → kernel says “OK, major = 250 has all 256 minors reserved.”

- Even if your driver only supports, say, 4 devices, the remaining 252 minor numbers are blocked (nobody else can use them).

  - They don’t consume RAM, but they consume the namespace.

- This is why it’s called wasted: the unused minors are unavailable to other drivers, even though you don’t use them.

👉 With alloc_chrdev_region(), you can instead request only what you need (e.g., count=4 minors), so no namespace waste.

Kobject (Kernel Object)

- A kobject is a fundamental building block in the Linux device model that represents kernel objects like devices, drivers, buses, and classes in a structured way.

- It provides reference counting (using kref) to ensure safe creation and deletion of kernel objects without memory leaks.

- Each kobject is part of a hierarchy (with parent-child relationships) and usually gets exposed in sysfs under /sys/, making kernel internals visible to user space.

- Kobjects are embedded inside larger structures like struct device or struct cdev, which allows those higher-level objects to automatically gain sysfs representation.

- By maintaining a standard interface, kobjects act as the foundation of the Linux device model, ensuring consistent organization, lifetime management, and user-space visibility.

🔹 Kobject Attributes

- Attributes are files created under a kobject’s sysfs directory, allowing user space to read or write kernel object properties in a file-like manner.

- They are defined using struct attribute or struct kobj_attribute, and linked with callbacks (show for read, store for write).

- Example: For a network device’s kobject (/sys/class/net/eth0/), attributes like mtu, operstate, and address appear as readable/writable files.

- This mechanism enables user space → kernel space communication without writing special IOCTLs or syscalls, using just simple file operations (cat, echo).

- Thus, kobject attributes make the Linux device model highly extensible and user-friendly, exposing kernel data structures as dynamic, interactive sysfs entries.

Why it’s useful

- Provides safe, standardized communication between kernel and userspace.

- No need to write custom IOCTLs or /dev drivers for every parameter.

- Used heavily for debugging, monitoring, and configuration (e.g., power management, device tuning, thermal sensors, etc.).

1. struct file_operations

- This structure in Linux defines how your character device will behave when user-space applications perform system calls like open(), read(), write(), and close().

- Each field in this struct is a function pointer that points to your driver’s implementation of that operation.

- Example: If user does read(fd, buf, size), the kernel calls your driver’s my_read() function (set in .read = my_read).

- Without filling this struct, your driver won’t respond to file operations.

- It acts as the “API contract” between user-space and kernel driver.

2. cdev_init(), cdev_add(), cdev_del()

These are used to register your character device with the kernel.

- cdev_init() → Initializes a struct cdev object and links it with your struct file_operations. Think of it as “preparing the device’s behavior”.

- cdev_add() → Registers that initialized cdev with the kernel, making it available under the assigned major/minor number. Without this, your device is not active.

- cdev_del() → Removes/unregisters the character device from the kernel when unloading your module.

1. copy_from_user()

📌 What it is:

- copy_from_user() is a kernel function used to safely copy data from user space to kernel space.

- It prevents direct access to user-space pointers inside kernel code.

🔧 Prototype:

long copy_from_user(void *kernel_buffer, const void __user *user_buffer, unsigned long count);

to: Kernel-space buffer to store the data.

- from: User-space pointer.

- n: Number of bytes to copy.

"copy_from_user() is used to transfer data from user space to kernel space. It returns non-zero on failure (e.g., if the pointer is invalid)."

// Write

static ssize_t my_write(struct file *file, const char __user *user_buf, size_t len, loff_t *offset) {

if (len >= BUF_SIZE)

len = BUF_SIZE - 1;

if (copy_from_user(kernel_buffer, user_buf, len))

return -EFAULT;

kernel_buffer[len] = '\0';  // Null-terminate

printk(KERN_INFO "Received from user: %s\n", kernel_buffer);

return len;

}

2. copy_to_user()

📌 What it is:

- copy_to_user() is used to send data from kernel space back to user space safely.

- Ensures memory safety and prevents kernel from directly writing to user-space addresses.

🔧 Prototype:

long copy_to_user(void __user *user_buffer, const void *kernel_buffer, unsigned long count);

to: User-space pointer to store result.

- from: Kernel buffer.

- n: Number of bytes to copy.

copy_to_user() is used to send data from the kernel to a user-space variable. It ensures the destination is valid and avoids segmentation faults."

/ /Read

static ssize_t my_read(struct file *file, char __user *user_buf, size_t len, loff_t *offset) {

size_t data_len = strlen(kernel_buffer);

if (copy_to_user(user_buf, kernel_buffer, data_len))

return -EFAULT;

return data_len;

}

--------------------------------------------------------------------------------------------------------------------

What is ioctl()?

ioctl stands for I/O control, and it's a system call used in device drivers to send control commands between user space and kernel space — beyond just read() and write() operations.

static long my_ioctl(struct file *file, unsigned int cmd, unsigned long arg);

struct file *file – Pointer to the file structure representing the open device file.

cmd – the command number defined by _IO, _IOW, _IOR, or _IOWR.

arg – typically a pointer to user-space data to pass or receive.

Usage of ioctl():

#define IOCTL_SET_NUM _IOW(MAJOR_NUM, 0, int)

#define IOCTL_GET_NUM _IOR(MAJOR_NUM, 1, int)

static int value = 0;

static long my_ioctl(struct file *file, unsigned int cmd, unsigned long arg) {

switch (cmd) {

case IOCTL_SET:

if (copy_from_user(&value, (int __user *)arg, sizeof(int)))

return -EFAULT;

printk(KERN_INFO "IOCTL SET: %d\n", value);

break;

case IOCTL_GET:

if (copy_to_user((int __user *)arg, &value, sizeof(int)))

return -EFAULT;

printk(KERN_INFO "IOCTL GET: %d\n", value);

break;

default:

return -EINVAL;

}

return 0;

}

------------------------------------------------------------------------------------------------------------------------

What is a Tasklet?

A tasklet is a bottom-half mechanism in the Linux kernel used to defer work that should not be done in interrupt context.
Tasklets run after an interrupt handler, in softirq context, and are used for non-time-critical tasks.

✅ Why use Tasklets?

- Interrupt handlers must finish quickly → don't do heavy work there.

- Tasklets are scheduled from interrupt context, but executed later.

- They allow deferring processing while staying in kernel space.

Tasklet is a bottom-half mechanism used to defer work from interrupt context.

- Defined using DECLARE_TASKLET(name, func, data).

- Scheduled using tasklet_schedule(&tasklet).

- Canceled using tasklet_kill(&tasklet) in cleanup.

ISR (Interrupt Service Routine) will not do heavy work — it just schedules a tasklet.

The tasklet function handles the deferred processing.

#include <linux/module.h>

#include <linux/kernel.h>

#include <linux/interrupt.h>

#define IRQ_NO 1  // Typically keyboard IRQ (used for testing only)

// Tasklet function prototype

void my_tasklet_func(unsigned long data);

// Declare and initialize tasklet

DECLARE_TASKLET(my_tasklet, my_tasklet_func, 0);

// Tasklet Function (Bottom Half)

void my_tasklet_func(unsigned long data) {

pr_info("Tasklet executed: deferred work after interrupt\n");

}

// Interrupt Handler (Top Half)

irqreturn_t my_irq_handler(int irq, void *dev_id) {

pr_info("Interrupt Occurred: IRQ %d\n", irq);

// Defer work to tasklet

tasklet_schedule(&my_tasklet);

return IRQ_HANDLED;

}

// Module Init

static int __init my_tasklet_irq_init(void) {

int ret;

pr_info("Tasklet + IRQ module loading...\n");

// Request IRQ (shared = true)

ret = request_irq(IRQ_NO, my_irq_handler, IRQF_SHARED, "my_irq_tasklet", (void *)(my_irq_handler));

if (ret) {

pr_err("Cannot register IRQ %d\n", IRQ_NO);

return ret;

}

pr_info("IRQ %d registered successfully\n", IRQ_NO);

return 0;

}

// Module Exit

static void __exit my_tasklet_irq_exit(void) {

// Free IRQ and kill tasklet

free_irq(IRQ_NO, (void *)(my_irq_handler));

tasklet_kill(&my_tasklet);

pr_info("Tasklet + IRQ module unloaded\n");

}

module_init(my_tasklet_irq_init);

module_exit(my_tasklet_irq_exit);

MODULE_LICENSE("GPL");

MODULE_AUTHOR("Swarna");

MODULE_DESCRIPTION("Tasklet example triggered by IRQ");

------------------------------------------------------------------------------------------------------------------------

What is a Workqueue?

A workqueue is a mechanism in the Linux kernel used to schedule work to be executed in process context, instead of interrupt context.

🔧 Why Use Workqueues?

- Unlike tasklets, workqueues can sleep, block, or access user-mode memory.

- Suitable for tasks that may take longer or need to use functions that may sleep (e.g., file operations, memory allocation).

✅ Key Differences: Tasklet vs Workqueue

| Feature | Tasklet | Workqueue |
| --- | --- | --- |
| Context | Softirq (atomic) | Process context |
| Can Sleep | ❌ No | ✅ Yes |
| Blocking Ops | ❌ Not Allowed | ✅ Allowed |
| Execution | ASAP after interrupt | Can be delayed |
| Use Case | Quick deferral | Complex, slow, or blocking tasks |
| API | tasklet_schedule() | schedule_work(), queue_work() |

#include <linux/module.h>

#include <linux/kernel.h>

#include <linux/interrupt.h>

#include <linux/workqueue.h>

#define IRQ_NO 1  // Example IRQ (keyboard)

static struct work_struct my_work;

// Workqueue handler (bottom half)

void workqueue_func(struct work_struct *work)

{

pr_info("Workqueue: deferred work executed in process context\n");

}

// IRQ handler (top half)

irqreturn_t my_irq_handler(int irq, void *dev_id)

{

pr_info("IRQ %d triggered. Scheduling workqueue.\n", irq);

// Schedule bottom half using workqueue

schedule_work(&my_work);

return IRQ_HANDLED;

}

// Module init

static int __init workqueue_example_init(void)

{

int ret;

pr_info("Loading workqueue + IRQ module...\n");

INIT_WORK(&my_work, workqueue_func);

ret = request_irq(IRQ_NO, my_irq_handler, IRQF_SHARED, "my_workqueue_irq", (void *)(my_irq_handler));

if (ret) {

pr_err("Failed to register IRQ %d\n", IRQ_NO);

return ret;

}

pr_info("IRQ %d registered and workqueue ready\n", IRQ_NO);

return 0;

}

// Module exit

static void __exit workqueue_example_exit(void)

{

free_irq(IRQ_NO, (void *)(my_irq_handler));

cancel_work_sync(&my_work);  // Waits for work to finish or cancel it

pr_info("Workqueue + IRQ module unloaded\n");

}

module_init(workqueue_example_init);

module_exit(workqueue_example_exit);

MODULE_LICENSE("GPL");

MODULE_AUTHOR("Swarna");

MODULE_DESCRIPTION("Simple Workqueue example triggered by interrupt");

A workqueue runs in process context, allowing it to sleep or block.

- Initialized using INIT_WORK(&wq, func);

- Scheduled using schedule_work(&wq);

- Use cancel_work_sync() in exit to clean up.

------------------------------------------------------------------------------------------------------------------------

What is a Wait Queue?

A wait queue is a kernel mechanism used to put a process to sleep until a specific condition becomes true, usually signaled by an interrupt or another kernel thread.

✅ Why Use Wait Queues?

- To block a process (e.g., read() or ioctl() call) until data is ready.

- Prevent busy waiting (CPU spinning).

- Often used with hardware events, shared buffers, or producer-consumer logic.

Important APIs

DECLARE_WAIT_QUEUE_HEAD(wq);               // Declare a wait queue

wait_event(wq, condition);                 // Sleep until condition is true

wait_event_interruptible(wq, condition);   // Same but signal-safe

wake_up(&wq);                              // Wake up one or more sleepers

#include <linux/module.h>

#include <linux/kernel.h>

#include <linux/init.h>

#include <linux/wait.h>

#include <linux/sched.h>

#include <linux/kthread.h>

#include <linux/delay.h>

static DECLARE_WAIT_QUEUE_HEAD(my_queue);

static int condition = 0;

static struct task_struct *my_thread;

// Kernel thread that waits

int wait_thread_fn(void *data)

{

pr_info("Thread: Waiting for condition...\n");

// Sleep until 'condition' becomes non-zero

wait_event_interruptible(my_queue, condition != 0);

pr_info("Thread: Woken up! Condition = %d\n", condition);

return 0;

}

// Module init

static int __init waitqueue_example_init(void)

{

pr_info("Wait Queue module loaded\n");

// Start thread

my_thread = kthread_run(wait_thread_fn, NULL, "wait_thread");

// Simulate delay and then wake up

msleep(3000);

condition = 1;

wake_up(&my_queue);

return 0;

}

// Module exit

static void __exit waitqueue_example_exit(void)

{

pr_info("Wait Queue module unloaded\n");

}

module_init(waitqueue_example_init);

module_exit(waitqueue_example_exit);

MODULE_LICENSE("GPL");

MODULE_AUTHOR("Swarna");

MODULE_DESCRIPTION("Simple wait queue example");

------------------------------------------------------------------------------------------------------------------------

SIMPLE CHARACTER DRIVER:

#include <linux/init.h>

#include <linux/module.h>

#include <linux/fs.h>

#include <linux/uaccess.h>

#define DEVICE_NAME "mychar"

#define BUF_SIZE 1024

MODULE_LICENSE("GPL");

MODULE_AUTHOR("swarna");

MODULE_DESCRIPTION("Simple Character Driver");

static int major;

static char kernel_buffer[BUF_SIZE];

static int open_count = 0;

// Open

static int my_open(struct inode *inode, struct file *file) {

open_count++;

printk(KERN_INFO "Device opened %d time(s)\n", open_count);

return 0;

}

// Read

static ssize_t my_read(struct file *file, char __user *user_buf, size_t len, loff_t *offset) {

size_t data_len = strlen(kernel_buffer);

if (copy_to_user(user_buf, kernel_buffer, data_len))

return -EFAULT;

return data_len;

}

// Write

static ssize_t my_write(struct file *file, const char __user *user_buf, size_t len, loff_t *offset) {

if (len >= BUF_SIZE)

len = BUF_SIZE - 1;

if (copy_from_user(kernel_buffer, user_buf, len))

return -EFAULT;

kernel_buffer[len] = '\0';  // Null-terminate

printk(KERN_INFO "Received from user: %s\n", kernel_buffer);

return len;

}

// Close

static int my_release(struct inode *inode, struct file *file) {

printk(KERN_INFO "Device closed\n");

return 0;

}

// File operations structure

static struct file_operations fops = {

.owner = THIS_MODULE,

.open = my_open,

.read = my_read,

.write = my_write,

.release = my_release,

};

// Module init

static int __init mychar_init(void) {

major = register_chrdev(0, DEVICE_NAME, &fops);

if (major < 0) {

printk(KERN_ALERT "Failed to register device\n");

return major;

}

printk(KERN_INFO "Registered device '%s' with major number %d\n", DEVICE_NAME, major);

return 0;

}

// Module exit

static void __exit mychar_exit(void) {

unregister_chrdev(major, DEVICE_NAME);

printk(KERN_INFO "Unregistered device '%s'\n", DEVICE_NAME);

}

module_init(mychar_init);

module_exit(mychar_exit);

------------------------------------------------------------------------------------------------------------------------

Upper Half (Top Half) :

- The upper half is the interrupt service routine (ISR) in Linux.

- It is executed immediately when an interrupt occurs.

- Runs in interrupt context, so it must be very fast and non-blocking.

- Cannot call functions that sleep or perform long tasks.

- Typically used to acknowledge the interrupt and schedule the bottom half.

Bottom Half :

- The bottom half handles the deferred work after the ISR.

- It runs in softirq or process context, depending on the mechanism.

- Common types: Tasklet, Workqueue, and Softirq.

- Suitable for time-consuming or blocking operations.

- It is scheduled by the upper half using tasklet_schedule() or schedule_work().

int register_chrdev(unsigned int major, const char *name, const struct file_operations *fops);

int alloc_chrdev_region(dev_t *dev, unsigned baseminor,unsigned count, const char *name);