`Platform drivers:

What is a Platform Device?

- These non-discoverable on-chip peripherals are called Platform Devices in Linux.

- Example: On Raspberry Pi or ARM SoC, you already have a built-in UART controller, I2C controller, GPIO pins → all are platform devices.

🔹 How does Linux know about them?

- The kernel learns about these devices from:

  - Device Tree (DTB) (modern way)

  - Platform data (older way)

  - ACPI tables (PC-style systems)

- Example (Device Tree entry for UART):

🔹 What is a Platform Driver?

- A Platform Driver is a special Linux driver written to handle platform devices.

- It provides two main functions:

  - probe() → Runs when a device is matched. Used for initialization.

  - remove() → Runs when the device is removed. Used for cleanup.

🔹 How matching happens?

- The kernel matches device and driver using:

  - Name matching (old way, pdev->name)

  - Device Tree compatible string (modern way)

If device compatible = "arm,pl011" and driver supports "arm,pl011", then probe() is called.

🔹 Platform Driver Structure

Defined in <linux/platform_device.h>:

In Linux, a device driver is a piece of kernel code that allows the operating system and user applications to communicate with hardware. Without a driver, the OS cannot talk to hardware directly.

Linux devices are divided into two broad categories: discoverable devices and non-discoverable devices.

1️ Discoverable Devices

- Definition: A discoverable device is a device that can announce itself to the kernel or system without manual configuration.

- How it works: These devices are connected to a bus that supports automatic discovery.

- Examples of discoverable buses:

  - PCI / PCIe: Peripheral Component Interconnect

  - USB: Universal Serial Bus

  - SATA: Serial ATA storage devices

- Mechanism: When the system boots, the kernel scans the bus and reads each device’s configuration registers.

  - For PCI, each device has a Vendor ID and Device ID in its configuration space.

  - The kernel reads these IDs and knows exactly what device exists.

- Result: No need to tell the kernel manually; the kernel automatically detects the device, its type, and resources (like memory, IRQs).

- Example: If you plug a USB mouse, the kernel sees it, reads its descriptors, and loads the appropriate driver automatically.

2️ Non-Discoverable Devices (Platform Devices)

- Definition: A non-discoverable device cannot announce its existence. It’s usually part of the SoC (on-chip peripheral).

- Why: These peripherals do not have standard IDs or registers to tell the kernel, unlike PCI or USB devices.

- Examples: UART controllers, I2C controllers, SPI, GPIO, timers inside a Raspberry Pi, BeagleBone, or other embedded SoCs.

- Problem: If the kernel doesn’t know they exist, it cannot load a driver automatically.

- Solution: You must describe these devices manually in the Device Tree, platform data, or ACPI table, so the kernel knows:

  - Device base address

  - IRQ number

  - Compatible string (for driver matching)

- Platform Driver Role: The platform driver reads this information from Device Tree or platform data and initializes the device.

3️ Key Difference (Discoverable vs Non-Discoverable)

| Feature | Discoverable Device | Non-Discoverable Device |
| --- | --- | --- |
| Can kernel detect automatically? | Yes | No |
| Examples | PCI, PCIe, USB, SATA | UART, I2C, SPI, GPIO inside SoC |
| How kernel knows it exists | Scans bus and reads IDs | You provide info via Device Tree or platform data |
| Driver loading | Kernel can auto-load | Needs platform driver and manual description |
| Resource allocation | Kernel can read from device itself | Kernel must get from DT/board file |

Linux devices are divided into two broad categories: discoverable devices and non-discoverable devices.

1️.Discoverable Devices (Auto-Detectable Devices)

A discoverable device is a hardware component that can automatically inform the operating system or kernel about its presence and capabilities without any manual configuration. These devices are connected to buses that support automatic enumeration, such as PCI, USB, or SATA. Automatic enumeration is the process by which the kernel or operating system automatically detects all devices connected to a bus, identifies them, and gathers information about them without any manual configuration. During system boot or when hot-plugged, the kernel queries the device for identification information like Vendor ID, Device ID, or descriptors, allowing the kernel to recognize the device type, allocate necessary resources such as memory addresses, I/O ports, and IRQs, and load the correct driver automatically. making them plug-and-play and eliminating the need for the user to manually tell the kernel that the device exists. For example, when a USB mouse is plugged in, the kernel reads its Vendor ID and Product ID, automatically loads the HID driver, and the mouse works without any manual configuration.

Device Stores Its Own Info: Each discoverable device has built-in identification registers or descriptors (like Vendor ID, Device ID for PCI, USB descriptors) that tell the kernel what the device is.

Kernel Scans the Bus: During boot or hot-plug, the kernel scans the bus (PCI, USB, SATA, etc.) and reads each device’s information directly from these registers, allowing it to identify the device type and required resources.

Bus Provides Communication Path: The bus itself doesn’t store device info; it only provides the path for the kernel to communicate with the device and retrieve its identification data.

2️. Non-Discoverable Devices (Platform Devices)

A non-discoverable device is a hardware component that cannot automatically notify the kernel of its existence or provide identification information. These devices are typically on-chip peripherals integrated into a System-on-Chip (SoC), such as UART controllers, I2C/SPI interfaces, GPIOs, or timers. Since the kernel cannot detect them on its own, developers must explicitly provide device details using mechanisms like Device Tree, ACPI tables, or platform data, which include base memory addresses, IRQ numbers, and compatible strings. All non-discoverable devices are attached to a single generic “platform bus,” which acts as a virtual bus connecting all SoC peripherals. The platform driver registers with this bus, and the kernel uses the provided information to match the driver with the device, initialize it, and enable its functionality for use. Unlike discoverable devices that are plug-and-play, non-discoverable devices require a manual introduction to the system before they can function. For example, on a Raspberry Pi, the UART0 peripheral is described in the Device Tree with its memory address and IRQ, and the PL011 platform driver reads this information, sets up the registers and interrupts, and makes the UART available as /dev/serial0 for user programs.

#include <linux/module.h>          // Needed for all kernel modules

#include <linux/platform_device.h> // For platform driver APIs

#include <linux/of.h>              // For Device Tree matching support

// ------------------------------------------------------

// PROBE FUNCTION

// ------------------------------------------------------

// This function is called when the kernel finds a device

// in the Device Tree (or platform data) that matches this driver.

static int my_probe(struct platform_device *pdev)

{

pr_info("my_platform_driver: probe called for device: %s\n", pdev->name);

// Here we would normally:

// 1. Request I/O memory regions

// 2. Map registers using ioremap()

// 3. Register interrupts with request_irq()

// 4. Initialize hardware

// 5. Register character device or subsystem

return 0; // Success

}

// ------------------------------------------------------

// REMOVE FUNCTION

// ------------------------------------------------------

// This function is called when the driver is removed or

// the device is being detached.

static int my_remove(struct platform_device *pdev)

{

pr_info("my_platform_driver: remove called for device: %s\n", pdev->name);

// Here we would normally:

// 1. Free I/O memory regions

// 2. Unmap registers

// 3. Free IRQs

// 4. De-initialize hardware

// 5. Cleanup allocated memory

return 0;

}

// ------------------------------------------------------

// DEVICE TREE MATCH TABLE

// ------------------------------------------------------

// This tells the kernel which "compatible" strings from

// the Device Tree this driver supports.

static const struct of_device_id my_of_match[] = {

{ .compatible = "mycompany,mydevice" }, // must match DT node

{ /* sentinel */ } // terminator

};

MODULE_DEVICE_TABLE(of, my_of_match);

// ------------------------------------------------------

// PLATFORM DRIVER STRUCTURE

// ------------------------------------------------------

// This structure links our probe/remove with the kernel's

// platform driver framework.

static struct platform_driver my_platform_driver = {

.probe  = my_probe,   // Called when device matches

.remove = my_remove,  // Called when driver is removed

.driver = {

.name = "my_platform_driver",     // Name of driver

.of_match_table = my_of_match,    // DT compatibility table

},

};

// ------------------------------------------------------

// DRIVER REGISTRATION MACRO

// ------------------------------------------------------

// This macro automatically creates init/exit functions

// to register/unregister the driver with the kernel.

module_platform_driver(my_platform_driver);

// ------------------------------------------------------

// MODULE INFORMATION

// ------------------------------------------------------

MODULE_LICENSE("GPL");

MODULE_AUTHOR("Swarna");

MODULE_DESCRIPTION("Simple Example Platform Driver");

mydevice0: my_device@10000000 {

compatible = "mycompany,mydevice";

reg = <0x10000000 0x1000>;  // base address + size

interrupts = <5>;           // IRQ number

};

When Linux boots, it sees "mycompany,mydevice" in DTB → checks registered platform drivers → finds match → calls my_probe().

What is a Platform Driver?

A platform driver is a type of Linux kernel driver written specifically to manage non-discoverable devices. Since these devices cannot automatically tell the kernel about themselves, the driver relies on external configuration—typically provided through a Device Tree, ACPI table, or platform data—to know where the hardware resides, which interrupts to use, and what kind of device it is.

Platform drivers are common in SoC (System-on-Chip) designs, where peripherals like UARTs, GPIOs, SPI/I2C controllers, timers, and PWM modules are integrated into the chip itself. Unlike PCI or USB devices, these components cannot identify themselves, so the kernel cannot probe them automatically.

- Example: On a Raspberry Pi, the UART0 controller is a non-discoverable peripheral. The PL011 platform driver reads its base address and IRQ from the Device Tree, initializes the UART hardware, and exposes it as /dev/serial0 for user-space programs.

Platform Bus:

All non-discoverable devices are attached to a generic virtual bus in the Linux kernel called the platform bus. This is not a physical bus like PCI or USB; rather, it is a kernel mechanism that organizes and registers all SoC peripherals.

The platform bus plays three key roles:

- Device Registration: Every non-discoverable device is represented as a platform device. When the kernel boots, the platform device is registered with the platform bus.

- Driver Binding: Platform drivers register themselves with the platform bus. The bus matches drivers to devices based on compatible strings, device names, or IDs provided in the Device Tree.

- Resource Management: The platform bus helps drivers access hardware resources such as memory-mapped registers, IRQs, and DMA channels provided in the device configuration.

Platform Devices:

A platform device is the kernel representation of a non-discoverable hardware peripheral. It contains all the information the driver needs to initialize and operate the device:

- Memory Addresses: The physical addresses of registers.

- IRQ Numbers: Interrupt lines associated with the device.

- Device Name / Compatible String: Used by the kernel to match the device with the correct driver.

- Platform Data (Optional): Custom configuration or driver-specific parameters.

Platform devices can be declared statically in board files, or more commonly today, they are described in the Device Tree.

- Example (Device Tree snippet for UART0):

How It Interacts with a Platform Device

how a Linux platform driver communicates and coordinates with a platform device within the Linux device driver model.

The interaction happens through the Linux device model, as follows:

1. Matching

- The platform device is described (via device tree or code) with a name or compatible string.

- The platform driver is registered with the kernel, including its own name.

- The kernel matches the device and driver based on this information.

2. probe() Callback

- Once a match is found, the kernel calls the driver’s probe() function.

- The probe() function gets a pointer to the struct platform_device.

- The driver can access hardware resources like memory regions or IRQs from this structure.

3. Resource Management

- The driver uses fields like resource in the platform device to allocate and initialize the hardware.

4. remove() Callback

- When the device is removed or the module is unloaded, the kernel calls the remove() function.

- The driver releases allocated resources and cleans up.

5. Integration

- The struct device within platform_device ensures proper registration in the kernel's device model.

- The driver can expose sysfs entries, interact with power management, and coordinate with other subsystems.

DTS stands for Device Tree Source.
It’s a text-based file format used in Linux (especially for embedded systems) to describe hardware components present in a system.

It is used by the kernel to understand the hardware layout, resources, and configuration without hardcoding it in the driver or kernel code.

Key Points:

- Purpose
DTS files describe:

  - What hardware is present.

  - Where the hardware is located (memory addresses, IRQs).

  - How the hardware should be configured.

  - Properties like clocks, regulators, GPIOs, etc.

- Where it’s used

  - Embedded devices like ARM-based boards (Raspberry Pi, BeagleBone, etc.).

  - Systems where hardware is fixed and cannot be discovered dynamically (non-pluggable).

  - Kernel reads the compiled version of the DTS called DTB (Device Tree Blob) at boot time.

- Not actual code but configuration
It doesn't perform operations—it only provides metadata.

Why DTS (Device Tree Source) is needed

DTS is not just a file  it solves real problems in how the Linux kernel handles hardware, especially for embedded systems and non-discoverable devices.

Main Reasons why DTS is used:

1️.Hardware description is separated from the kernel code

- Without DTS, hardware details like memory addresses, interrupts, clocks would need to be hardcoded inside drivers or the kernel.

- This makes the kernel less portable and harder to maintain.

- With DTS, you describe the hardware in a configuration file, keeping it separate from the driver logic.

2️.Supports multiple boards without changing drivers

- The same driver can work across different boards by simply changing the DTS.

- For example, a driver for a display controller can be used on many boards by adjusting the configuration in the DTS.

3️.Essential for non-discoverable hardware

- Some hardware interfaces like GPIO, legacy devices, or custom embedded peripherals do not support automatic discovery like PCI or USB.

- DTS provides a structured way to inform the kernel about these devices and their resources (addresses, IRQs).

HOW DOES KERNEL MATCHES DEVICE WITH DRIVER

Matching in Device Tree Based Platforms – Platform Bus Role

- The kernel parses the device tree and creates platform device instances.

- These devices are added to the platform bus.

- The driver registers itself with the platform bus using platform_driver_register().

- It reads the compatible property from the device tree.

- It compares it with entries in the driver's of_match_table.

- The bus’s internal matching function (platform_match() or similar) compares:

  - The device’s compatible property → listed in of_match_table.

- If a match is found:

  - The bus calls the driver’s probe() function.

The platform bus is responsible for orchestrating this process, even though the comparison logic involves device tree fields.


1. How does the Linux kernel detect and register a platform device during boot?

When the kernel parses the Device Tree Blob (DTB) at boot, it creates platform device structures for every node containing a “compatible” string. These are registered under the platform bus. Later, when the corresponding platform driver registers itself, the kernel matches it with the device using this compatible property and calls probe().

2. What actually happens inside the probe() function of a platform driver?

The probe() function is invoked once the device-driver match occurs. It retrieves hardware details from the Device Tree using APIs like of_property_read_*(), maps register memory via ioremap(), requests interrupts, and initializes the hardware. It’s the main entry point that prepares the device for use by the OS or user space.

3. How is a Device Tree used to make platform drivers portable across boards?

Instead of hardcoding hardware details like base addresses, IRQs, or GPIOs, these are described in the Device Tree (.dts/.dtsi). The driver reads them dynamically, making the same driver binary reusable across multiple boards. Only the Device Tree file changes per board — not the driver source code.

4. What is the role of the of_match_table in platform drivers?

The of_match_table contains the list of supported “compatible” strings for a driver. During boot, the kernel compares each platform device’s compatible string against this table. When a match occurs, the platform bus automatically triggers the driver’s probe() method — ensuring correct driver-device binding.

5. What would happen if the compatible string in DTS doesn’t match the driver?

If the compatible property in the DTS node doesn’t match any string listed in the driver’s of_match_table, the kernel will never call the driver’s probe() function. The device node will still exist, but the driver won’t bind — causing initialization failure, visible in dmesg as “no compatible driver found.”

6. Why is ioremap() used in platform drivers?

Platform devices use memory-mapped I/O for hardware register access. ioremap() converts the physical address (from DTS “reg” field) into a virtual address accessible by the kernel. Without it, accessing peripheral registers directly would cause faults because the kernel cannot access raw physical memory safely.

7. How do you safely release hardware resources in a platform driver?

Resources acquired during probe() — like I/O memory regions, interrupts, and buffers — must be freed in remove(). Functions like iounmap(), release_mem_region(), and free_irq() ensure proper cleanup. This prevents resource leaks and allows clean reloading or unbinding of the driver module.

8. What role does the platform bus play in Linux driver architecture?

The platform bus acts as a logical connector between all SoC peripherals (devices) and their corresponding drivers. It performs driver-device matching using names or compatible strings, manages resource ownership, and calls the respective probe/remove callbacks. It provides a unified interface for all non-discoverable devices.

9. What is the significance of the module_platform_driver() macro?

This macro simplifies driver registration by automatically generating init and exit routines. It registers the driver with the kernel’s platform bus when the module loads and unregisters it on removal. It reduces boilerplate code, making the module concise and less error-prone.

10. How do you debug why a platform driver’s probe() isn’t being called?

First, check if the device node exists in /sys/firmware/devicetree/base/. Verify the compatible string matches the driver’s of_match_table. Use dmesg | grep <driver> to trace probe activity. If missing, recheck DTS syntax, module name, and ensure the driver is loaded (lsmod) and not blacklisted.

11. What are common mistakes developers make in DTS-driver integration?

Typical errors include incorrect “compatible” strings, mismatched base addresses, or missing status = "okay" property in DTS nodes. Developers sometimes hardcode values instead of using of_property_read_*() APIs. Such mismatches prevent device binding or lead to probe failures visible in kernel logs.

12. How can interrupts be handled inside a platform driver?

The driver reads the interrupt number from DTS using platform_get_irq() and registers an ISR with request_irq(). Inside the ISR, you handle device-specific events or data readiness. Always ensure proper cleanup with free_irq() in remove() to prevent dangling interrupt handlers.

13. Why is the Device Tree preferred over static platform data?

Earlier kernels used static “platform_data” in C source files, making hardware info tied to board-specific code. The Device Tree separates hardware configuration from the kernel code, improving reusability. Developers can now add new hardware support without recompiling the kernel — just by updating the DTS.

14. How do power management and platform drivers interact?

Platform drivers can implement suspend and resume callbacks (.suspend and .resume) to handle low-power transitions. When the system enters sleep mode, these callbacks disable clocks or interrupts safely, and upon resume, they reinitialize hardware. This integration ensures efficient power usage in embedded systems.

15. What practical debugging steps do you follow when a platform device misbehaves?

Enable dynamic debugging using echo "file driver.c +p" > /sys/kernel/debug/dynamic_debug/control to trace runtime logs. Verify DTS correctness, inspect /sys/class/ or /sys/devices/platform/ for device registration, and check dmesg for probe errors. Using dev_info() prints tagged logs with device context, making tracing easier.

### Matching in Device Tree Based Platforms – Role of the Platform Bus

In Linux, for device tree–based embedded systems, the platform bus acts as the key coordinator that connects devices (hardware) defined in the Device Tree Source (DTS) with their corresponding platform drivers in the kernel. The entire matching process is carefully orchestrated by the kernel’s platform bus subsystem, which ensures that each hardware component is properly bound to its controlling driver.

- When the system boots, the kernel parses the Device Tree Blob (DTB), which describes all hardware components available on the board — including their compatible strings, register addresses, interrupts, and other properties.

- For every hardware node described under the Device Tree that corresponds to a platform device, the kernel creates a struct platform_device instance in memory.

- Each such platform device structure contains essential fields like:

  - Device name

  - Resource list (memory, IRQ, I/O regions)

  - Device Tree node pointer (of_node)

  - compatible property information.

- These platform device instances are then registered with the platform bus using internal kernel routines like of_platform_populate().
This effectively adds the devices to the platform bus device list.

- Meanwhile, platform drivers (written by developers) are registered with the same platform bus by calling the function
platform_driver_register() inside their module_init() function.

- Each platform driver provides an of_match_table, which is an array of struct of_device_id.
This table lists all compatible strings that the driver supports — for example:

static const struct of_device_id my_driver_of_match[] = {

{ .compatible = "vendor,my-device" },

{ /* sentinel */ }

};

- When a driver is registered, the platform bus internally stores this information and waits to check if any existing platform devices can be paired with it.

- The platform bus now plays the role of a mediator — it tries to match each platform device with a driver by comparing their properties.

- The core matching logic is handled by a function like platform_match(), which is part of the platform bus driver model.

- platform_match() checks several conditions, in this order:

  - If the device was described via device tree, it reads the compatible string from the device node (of_node->compatible).

  - It compares that string with all entries in the driver's of_match_table.

  - If a match is found, it confirms that this driver supports this specific hardware.

- This matching process ensures that only the correct driver binds to the intended hardware device — avoiding incorrect or duplicate bindings.

- Once a match is confirmed, the platform bus automatically calls the driver’s probe() function.

- The probe() callback receives a pointer to the matched struct platform_device, through which the driver can access:

  - Memory-mapped I/O addresses

  - Interrupt numbers

  - Any other platform-specific configuration.

- Inside the probe() function, the driver typically performs:

  - Memory mapping (devm_ioremap_resource())

  - IRQ registration (devm_request_irq())

  - Device initialization and hardware bring-up.

- The platform bus ensures synchronization and ordering, meaning that the probe() is called only after both the device and the driver are available.

- If at any point the device is removed (hot-unplugged) or the driver is unloaded, the platform bus again coordinates the cleanup by calling the driver’s remove() function.

- Throughout this process, the platform bus maintains the relationship between struct platform_device and struct platform_driver using the kernel’s internal struct device framework.

- Even though the actual comparison happens based on the Device Tree’s compatible field and the driver’s of_match_table, the platform bus is the component responsible for managing the interaction and invoking the correct callbacks.

- In other words, the bus doesn’t define what matches — it defines how matching is handled, how probe() is triggered, and how the two sides communicate through the Linux device model.

- The result is a modular, scalable, and hardware-independent framework, where new devices can be supported simply by updating the Device Tree and adding a matching entry in the driver’s of_match_table.

- Hence, the Platform Bus acts like a middleman or coordinator that connects the static hardware descriptions in the Device Tree with dynamic driver registrations in the kernel, enabling smooth plug-and-play style binding in embedded Linux systems.

In short:

The “Platform Bus” in Linux is the backbone of driver-device binding on Device Tree–based systems. It takes devices parsed from the DTB, compares their “compatible” strings with the driver’s match table, and once matched, triggers the driver’s probe function to initialize and manage the actual hardware.

When a Linux system boots on an embedded board, the kernel doesn’t automatically know what hardware exists because SoC peripherals (like UART, SPI, GPIO) are not discoverable. So the Device Tree Source (DTS) is used to describe all hardware connected to the CPU. The bootloader loads both the kernel and the Device Tree Blob (DTB) into memory, and the kernel reads this DTB during boot.

The kernel parses the DTB and creates internal data structures (struct device_node) for each hardware node described in the DTS. Then, using functions like of_platform_populate(), the kernel walks through those nodes and creates a platform device for each one. A platform device is represented by the structure struct platform_device ,it stores information about the hardware like register address ranges, IRQ numbers, and the name of the device. This happens very early in the boot process (long before any driver code runs).

Later, when you load your driver (either built-in or via insmod), it registers itself with the kernel using a struct platform_driver. This structure contains pointers to functions like .probe, .remove, and an embedded .driver field which includes the of_match_table  a list of supported compatible strings from the Device Tree. The driver registration is done through a call like platform_driver_register(). Once registered, the driver is added to the kernel’s list of available platform drivers.

Now, the kernel has both sides ready:

- platform devices (created from DTS), and

- platform drivers (registered by your driver code).

The kernel’s platform bus subsystem automatically tries to match each device to a driver. The matching happens primarily using the Device Tree compatible string — if the device’s compatible value matches one of the entries in the driver’s of_match_table, the kernel calls that driver’s probe() function. If the device was not described by a Device Tree, the kernel can also match based on the device’s .name and the driver’s .driver.name.

Once the probe() function is called, your driver can use the struct platform_device *pdev pointer to get resources from the DTS — like memory addresses and interrupts — using APIs such as platform_get_resource() or of_get_named_gpio(). Inside probe(), you usually initialize the hardware (ioremap registers, request IRQs, register subsystems, etc.).

When the driver is removed, the kernel calls the driver’s .remove() callback to release resources. The platform device itself remains in memory until shutdown, because it represents the hardware that always exists on the board.

👉 In short:

- DTS describes the hardware (compatible, reg, irq).

- Kernel reads it at boot and creates platform devices.

- Driver registers as a platform driver with an of_match_table.

- Kernel matches them via compatible → calls probe().

- The driver initializes and controls the hardware.