Power Management Integrated Circuits (PMICs):

A power management integrated circuit (PMIC) is used to manage power on an electronic devices or in modules on devices that may have a range of voltages.

A semiconductor component that manages and optimizes the power supply within electronic devices.

A PMIC (Power Management Integrated Circuit) is a specialized electronic chip that controls and manages the distribution of electrical power within a device. It regulates voltage levels, current flow, and power sequencing to ensure that each component receives stable and appropriate power for optimal performance. PMICs are commonly used in battery-powered and portable devices such as smartphones, tablets, laptops, and wearable technology, where efficient power management is crucial. By integrating multiple power functions—like voltage regulation, battery charging, and power switching—into a single chip, PMICs reduce the overall size and complexity of the device’s power system. This integration helps extend battery life, minimize energy loss, prevent overheating, and improve the reliability of the electronic system. Overall, PMICs play a vital role in enhancing energy efficiency and ensuring smooth operation of modern electronics.

Key functions of a PMIC:

1.Voltage Regulation: Converts and regulates voltage levels to provide the correct power to different subsystems using components like DC/DC converters.

A DC-DC converter is an electronic device that changes a direct current (DC) voltage from one level to another, either by raising or lowering the voltage, or both.

Common Types :

Types of Voltage Regulators in PMICs

PMICs usually integrate multiple types of voltage regulators:

- Buck Converters (Step-Down Regulators): Convert a higher input voltage to a lower output voltage efficiently.

- Boost Converters (Step-Up Regulators): Increase the input voltage to a higher output voltage.

- Step-up/Step-down (Buck-Boost) Converter:Can either increase or decrease the input voltage.

- LDO Regulators (Low Dropout Regulators): Provide a stable, low-noise output voltage with minimal difference between input and output voltage.

- Charge Pumps: Use capacitors to increase or invert voltage without inductors (less common in this kind of PMIC).

2. Battery Charging and Management: Handles the charging process for rechargeable batteries, such as lithium-ion batteries, and monitors their status.

One of the key functions of a PMIC is Battery Charging and Management, especially in devices that run on rechargeable batteries like smartphones or tablets.

Basically, the PMIC is responsible for making sure the battery is charged safely, efficiently, and correctly.

It controls the charging process, which usually happens in stages. First, if the battery is very low, it starts with a slow pre-charge. Then it moves to a constant current phase, and finally a constant voltage phase, and it stops charging when the battery is full. This prevents overcharging, which can damage the battery.

The PMIC also supports different battery types like lithium-ion or lithium-polymer, because each type has its own safe voltage and current limits.

To make sure everything is safe, the PMIC monitors things like battery temperature, voltage, and current. If anything goes out of range — like if the battery is too hot or if the voltage is too high — it can stop charging immediately to prevent damage or danger.

Another important job is fuel gauging. That means it estimates how much charge is left in the battery — like when your phone shows 50% battery. It can also track battery health and how many times it’s been charged.

The PMIC also detects where the power is coming from — like a USB cable or a wall charger — and adjusts the charging speed depending on the source. If it's a fast charger, it can charge quicker, but it always stays within safe limits.

In devices with multiple battery cells, it also does cell balancing, making sure all cells charge evenly. This helps the battery last longer and perform better.

Lastly, the PMIC can communicate with the main processor, so the system can show battery info to the user or take action like turning off the device if the battery is critically low.

So overall, battery charging and management is not just about charging — it's about charging smartly, safely, and extending battery life."
The PMIC manages battery charging in a safe and efficient way. It controls the charging stages, protects against overvoltage or overheating, and adjusts charging speed depending on the power source. It also tracks battery health and level using fuel gauging and even balances cells in multi-cell batteries. In short, it makes sure the battery charges properly and lasts longer, without damaging the device."

3.Power Sequencing: Controls the order in which different components are powered on or off to ensure stable operation and prevent damage.

Another important function of the PMIC is Power Sequencing. This refers to how the PMIC controls the order in which different parts of a system are turned on or off.

In complex electronic devices like smartphones, tablets, or embedded systems, there are many different components — like the processor, memory, sensors, display, and so on. Each of these components might require power at different times and in a specific sequence.

If everything is powered up randomly, it can lead to unstable behavior, voltage spikes, or even damage to sensitive components. So, power sequencing ensures that everything turns on in the correct order, with the right timing and delays between each stage.

For example, in most systems, the core voltage for the processor must be applied before the I/O voltage. Similarly, some chips need to be powered only after others are fully stable.

The PMIC handles all of this automatically by using internal timers and control logic. It can also manage the power-down sequence to make sure components shut off in the right order, which is just as important to avoid data corruption or damage.

In summary, power sequencing by the PMIC makes sure the system powers up and down in a controlled and safe way, which is crucial for system stability and long-term reliability.

### What Is a Ramp Rate?

Ramp rate is how fast the voltage goes from 0V to its final value when a power rail is turned on (or from the final value to 0V when turning off).

### 1️.Simultaneous Power-Up/Down

All voltages turn ON or OFF at the same time.

All power rails use the same enable signal.

All voltages ramp up together — same speeed.

May cause a large current surge, because many parts turn on together.

A "large current surge" refers to a sudden and significant increase in electric current that occurs when multiple power rails or components turn on simultaneously. This can cause a brief but potentially problematic spike in power demand, potentially leading to:

1. Power supply strain

2. Voltage fluctuations

3. Increased heat generation

4. Potential system instability or even damage.

In the context of simultaneous power-up/down, the simultaneous activation of multiple components can draw a large amount of current, causing a surge.

In simultaneous power sequencing, all voltage rails are turned on or off at the same time using a single enable signal. This means all power supplies begin ramping up their output voltages together, typically at the same rate. While this method is very simple to implement and offers a fast startup time, it comes with the drawback of causing large inrush currents. Since all the power rails start charging their respective capacitive loads simultaneously, the power supply might experience stress, potentially affecting reliability. Furthermore, if the load on each rail is uneven, this method could cause voltage overshoots or undershoots, which might damage sensitive components. Due to these risks, simultaneous sequencing is best suited for simple or low-power systems like basic microcontroller-based boards, where components are not dependent on a strict power-up order.

### 2️.Sequential (Ordered) Power-Up/Down

In contrast, sequential power sequencing involves powering on each voltage rail one after the other in a specific, predefined order. The next rail in the sequence only begins ramping up after the previous one has reached and stabilized at its target voltage. This method is critical for complex systems such as FPGAs, SoCs, processors, and DDR memory, where the internal logic requires certain voltages — like core voltages — to be stable before others — such as I/O or memory interfaces. Sequential sequencing allows for individual control of ramp rates and delays between rails, which reduces electrical stress, minimizes inrush current, and protects sensitive components. Although it takes longer to power up and demands more complex hardware control, such as a Power Management IC (PMIC) or external sequencing logic, it is essential for ensuring safe and reliable system operation in advanced designs.

### 3️.Ratiometric or Margining Sequencing

The third method, ratiometric sequencing, focuses on maintaining a proportional voltage relationship between multiple power rails as they ramp up or down. Instead of sequencing rails one after another or switching them all on simultaneously, ratiometric sequencing ensures that all voltages rise and fall together, but their ratios remain fixed throughout the process. For example, if one voltage rail is 3.3V and another is 1.65V (a 2:1 ratio), this ratio is preserved during the entire ramp-up and ramp-down period. This technique is especially important in analog, RF, and mixed-signal circuits, where maintaining voltage balance between supply rails is critical to avoid noise, distortion, or signal integrity issues. Because it requires highly synchronized ramp rates and precise voltage control, ratiometric sequencing is more complex to implement and usually requires specialized PMICs or analog power controllers. However, in systems where voltage mismatches could lead to performance degradation, this method is the most effective.

### Final Thoughts:

To summarize, the choice of power sequencing method depends on the system’s complexity, sensitivity, and performance requirements. Simultaneous sequencing is ideal for simple applications where speed and ease are important, but it may pose risks in terms of inrush current and power rail interaction. Sequential sequencing is necessary for complex digital systems with strict power dependencies, ensuring safe startup and shutdown. Ratiometric sequencing, while more technically demanding, is best suited for high-performance analog and RF systems where maintaining stable voltage ratios is essential. Understanding and implementing the right power sequencing method ensures system stability, protects components, and enhances long-term reliability.

4.Load Switching: Manages the distribution of power to various loads or circuits within a system.

Another key function of the PMIC is Load Switching. This refers to how the PMIC manages the distribution of power to different parts of a system — which we call 'loads'. A load can be anything that consumes power, like the CPU, display, sensors, or wireless modules.

Instead of supplying power to all components all the time, the PMIC uses internal switches — often called load switches — to turn power on or off to each part of the system as needed.

This is really important for power efficiency. For example, if the camera or Bluetooth module isn't being used, the PMIC can switch off power to that section completely to save battery. When those parts are needed again, it quickly restores power.

Load switching also helps with thermal management, because unused parts of the system aren't drawing current and generating heat.

Another benefit is protection. If there’s a short circuit or an overload on one output, the PMIC can shut down just that load without affecting the rest of the system. This helps prevent damage and keeps the rest of the device running normally.

So in simple terms, load switching is how the PMIC controls which components receive power and when, helping improve battery life, system reliability, and safety."

Load Switching means the PMIC controls power delivery to different parts of the system — like turning off unused components to save battery. It uses internal switches to connect or disconnect power to each circuit. This helps with energy efficiency, reduces heat, and protects the system from faults like short circuits."

5.Power Path Control: Seamlessly switches power delivery between different sources like a wall adapter, USB port, and battery.

Power Path Control is a very important function of a PMIC that manages how power flows from different sources to the device and battery.

Many devices can be powered or charged from multiple sources — like a wall charger, a USB port, or the internal battery. The PMIC’s job is to seamlessly switch between these sources without interrupting the device’s operation.

For example, when you plug in your phone charger, the PMIC automatically directs power from the charger to run the device and charge the battery at the same time. If you unplug the charger, it instantly switches power delivery to the battery without any downtime or reset.

This switching happens smoothly to avoid power interruptions, which could cause the device to crash or reset.

Additionally, power path control makes sure that the battery isn’t drained unnecessarily when an external power source is available, and it protects the system by preventing backflow of current from the battery to the power source.

So, in short, Power Path Control intelligently manages where the device gets its power from and ensures uninterrupted, efficient, and safe power flow between chargers, USB, and battery."

Power Path Control lets the PMIC switch power smoothly between different sources like a wall charger, USB, or battery. This means when you plug or unplug the charger, the device keeps running without interruption. It also protects the battery and system by managing the power flow safely."

Benefits of using PMICs

- Space Saving: Integrates multiple functions onto a single chip, which is crucial for space-constrained devices.

- Cost Reduction: Decreases the number of individual components needed, which lowers the bill of materials (BoM) cost.

- Improved Efficiency: Optimizes power usage to extend battery life and reduce overall system power consumption.

- Simpler Design: Reduces the complexity of the printed circuit board (PCB) and overall system design.

- Enhanced Stability: Provides feature

Introduction to PMICs

Power Management Integrated Circuits, commonly known as PMICs, are essential components in modern electronic systems. As embedded systems become more complex and power-hungry, managing power delivery efficiently has become a crucial part of hardware and software design. A PMIC is a system-on-chip solution that consolidates multiple power functions—such as voltage regulation, power sequencing, battery charging, and system monitoring—into a single IC. Instead of using discrete components for each function (e.g., separate LDOs and buck converters), a PMIC combines all these into a unified package, resulting in significant board space savings, improved efficiency, and better control over system power states.

PMICs are particularly important in automotive, mobile, industrial, and IoT applications where space, efficiency, and precise control over power rails are critical. The PMIC acts as the power backbone of the system, ensuring that different components—processors, memory, sensors, displays, and peripherals—receive the correct voltage at the right time, and in the right sequence.

2.PMIC Functional Overview

A typical PMIC, such as the TPS6521905-Q1 from Texas Instruments (used in your project), is a multi-function device capable of delivering regulated power to various parts of the system. It usually includes multiple buck converters (for high-efficiency step-down voltage regulation), LDOs (for noise-sensitive analog circuits), and sometimes boost converters (for step-up voltages). It also contains control logic for power sequencing—essential for components that must be powered up or down in a specific order to avoid hardware damage or data corruption.

PMICs often feature an interface like I2C or SPI, which allows a host processor (such as an ARM-based SoC running Linux) to communicate with the PMIC for dynamic configuration. Through this interface, voltages can be adjusted at runtime, regulators can be enabled or disabled, faults can be read, and system power states can be controlled. Additionally, PMICs may provide monitoring features such as over-voltage protection (OVP), under-voltage lockout (UVLO), thermal shutdown, and watchdog timers that help maintain the stability and reliability of the system.

3. Role of the PMIC in Embedded Linux Systems

In a Linux-based embedded system, the PMIC must be integrated into the power management subsystem of the Linux kernel. This requires writing a kernel driver that interfaces with the PMIC over I2C or SPI. The driver allows the operating system to recognize and interact with the PMIC's features, such as enabling/disabling voltage regulators, changing voltage levels dynamically (e.g., during CPU frequency scaling), and handling fault conditions.

Linux has a well-established regulator framework, which provides a standard interface for controlling power supplies. When writing a PMIC driver, each regulator channel (e.g., BUCK1, BUCK2, LDO1, etc.) is registered with the regulator framework, allowing other parts of the kernel (like device drivers for the CPU, GPU, or peripherals) to request and control power from those regulators. This abstraction is important for achieving modularity, reuse, and efficient power management.

Importance in Automotive and Safety Applications

Since TPS6521905-Q1 is an automotive-grade PMIC (indicated by the Q1 designation), it must meet strict standards for reliability, fault tolerance, and diagnostics. The Linux driver must also meet higher software quality standards—especially in functional safety systems. Watchdog functionality is especially critical in these contexts. If the system hangs, the watchdog timer (configured via the PMIC) ensures the device is automatically reset and recovers to a known-good state.

In systems using frameworks like ASIL (Automotive Safety Integrity Level) or ISO 26262, proper power management and fault monitoring via PMICs is a critical requirement.

Conclusion

In summary, PMICs are one of the most critical components in an embedded system’s power architecture. Developing a Linux kernel driver for a PMIC like TPS6521905-Q1 involves a deep understanding of hardware (from the PMIC datasheet), software (Linux kernel subsystems like I2C and regulators), and configuration (Device Tree and user-space tools). By exposing the PMIC’s features to the Linux kernel in a structured, modular way, developers ensure robust, efficient, and safe power management across the entire system. This is especially important in mission-critical applications like automotive electronics, where system uptime, safety, and reliability are non-negotiable.

What is the Power-Up Sequence?

The power-up sequence is the order and timing in which the PMIC enables its multiple voltage rails (power supplies) when the system is turned on.

Each block of an SoC (CPU, GPU, memory, peripherals, etc.) needs to receive power at the right voltage level and in the correct order, otherwise, the system may:

• Fail to boot,

• Draw excessive current,

• Or even be permanently damaged.

Typical Stages of a PMIC Power-Up Sequence

1. Power Source Detection

• The PMIC first detects a valid input source:

◦ Battery voltage (VBAT)

◦ USB or DC input (VDD_MAIN)

• It verifies that the voltage is within safe limits using undervoltage lockout (UVLO) and overvoltage protection (OVP).

2. Power-On Trigger

• The PMIC starts the sequence when:

◦ A power button is pressed (PWRON signal),

◦ Or an external enable pin (EN) or system control signal is received,

◦ Or a watchdog/RTC alarm event occurs (for timed wake-up).

3. Reference and Bias Generation

Bandgap Reference Circuit starts

• The PMIC turns ON its bandgap reference, a temperature-stable voltage source (typically around 1.2 V).

• This reference voltage is the “gold standard” inside the PMIC that all regulators (like LDOs and DC-DC converters) compare against.

• Internal Bias Circuits activate

• These generate the bias currents and voltages that power internal control blocks — comparators, amplifiers, oscillators, and protection circuits.

• Think of them as the “internal power rails” for the PMIC’s own logic and control units.

• Oscillator starts running

• The internal oscillator (or sometimes an external clock) starts oscillating to provide the timing for switching regulators.

• Switching converters rely on this clock to control their duty cycles and maintain stable output voltages

Step 4: System Power State Machine Starts

What happens:

1. Internal state machine activates:

◦ The PMIC has a built-in logic controller (like a tiny microcontroller) that executes a predefined startup sequence.

◦ This sequence is stored in PMIC firmware or hardwired logic.

2. Regulator control:

◦ Based on the sequence, the state machine decides when and in which order each output regulator (BUCKs or LDOs) should turn ON.

◦ It can also insert delays between regulators to prevent voltage spikes or inrush currents.

Why this step is important:

• Correct power-up order:

Some components (like CPUs or memory) can be damaged if powered before their supply rails are stable.

• System stability:

Ensures no part of the system tries to operate before its voltage is ready.

• Prevents glitches:

Avoids sudden voltage drops or overshoots that can reset or corrupt devices.