Device Tree & Log Analysis in Linux BSP Development

### Why do we need a BSP (Board Support Package)?

- Bridges Hardware and Software

  - A BSP acts as the bridge between the raw hardware (CPU, peripherals, memory, devices) and the operating system (like Linux, RTOS, or Android).

  - Without BSP, the OS would not know how to communicate with board-specific devices.

- Board-Specific Customization

  - Every hardware board is different (different CPU, different RAM layout, different peripherals).

  - BSP provides custom drivers, startup code, and device tree entries that make the OS run correctly on that specific board.

- Smooth OS Boot-up

  - During boot-up, BSP initializes the processor, clocks, memory controller, and device interfaces.

  - It ensures the OS kernel loads properly and knows exactly where and how to access hardware resources.

- Accelerates Development

  - Instead of writing everything from scratch, developers use BSP as a ready-to-use foundation.

  - It reduces time to bring up new hardware and allows engineers to focus on application development rather than low-level setup.

- Enables Portability & Reuse

  - With BSP, the same OS can be ported to different boards by just modifying BSP, without rewriting the whole OS.

  - This makes hardware upgrades and product variations easier.

- Ensures Hardware-Software Stability

  - BSP is carefully tested for compatibility with the board, ensuring stable and reliable communication between hardware and OS.

  - It avoids system crashes due to misconfigured peripherals.

In short (presentation line):
A BSP is like a translator—it allows the operating system and the hardware board to understand each other, ensuring smooth boot-up, stability, and faster product development.

## Problem Statement

“Normally, when we develop an embedded Linux system:

- The device tree may not describe our custom hardware properly.

- Debugging becomes difficult because after a crash, kernel logs are lost.

- Manual log checking is slow and error-prone.

So, the problem is:
How can we extend the BSP to support new hardware,
Ensure logs persist after crashes,
And provide an easier way to detect errors automatically?”

## 🔹 3. Objectives

“Our objectives were:

- Modify and extend .dts and .dtsi files for custom hardware.

- Implement a kernel driver that reads properties from the device tree.

- Enable and configure pstore for persistent crash log capture.

- Collect and analyze logs using dmesg and journalctl.

- Automate basic error detection using scriptable filters.”

## . Reading Values from DTS using of_property_read_*

“When we write a kernel driver, the device tree (DTS) provides hardware configuration details like base address, IRQ numbers, GPIO pins, clock frequencies, etc.

The driver can read these using the Device Tree APIs such as of_property_read_u32, of_property_read_string, etc.”

### Example DTS Entry:

mydevice@1000 {

compatible = "myvendor,mydevice";

reg = <0x1000 0x100>;

irq = <42>;

label = "TestDevice";

};

### Example Driver Code:

static int my_probe(struct platform_device *pdev)

{

struct device_node *np = pdev->dev.of_node;

u32 irq_num;

const char *label;

if (of_property_read_u32(np, "irq", &irq_num) == 0)

pr_info("IRQ Number from DTS: %d\n", irq_num);

if (of_property_read_string(np, "label", &label) == 0)

pr_info("Label from DTS: %s\n", label);

return 0;

}

Explanation for Presentation:

- of_property_read_u32 reads integer properties (like IRQ number).

- of_property_read_string reads string properties (like label).

- The values are printed to kernel logs (visible in dmesg).

This makes the driver flexible because hardware parameters are taken from DTS instead of hardcoding in C code.

## 🔹 2. Using journalctl for Log Analysis

“Next, once our driver is loaded and prints messages, we need a reliable way to check system logs.

In Linux with systemd, the main log viewer is journalctl.”

### Key Commands:

# Show all logs

journalctl

# Show only kernel logs

journalctl -k

# Show recent logs

journalctl -xe

# Show logs since boot

journalctl -b

Explanation for Presentation:

- Unlike dmesg, which only shows the current kernel buffer, journalctl stores logs persistently on disk.

- You can search specific driver logs:

journalctl -k | grep mydevice

- Useful for debugging because logs survive reboot, unless explicitly cleared.

## 🔹 3. Persistent Crash Logs with pstore

“Finally, what happens if the system crashes?

- Normal logs are lost after reboot.

- To solve this, Linux provides pstore (Persistent Storage).

It stores crash logs in a reserved memory area or flash, so after reboot, we can recover them.”

### Setup:

- Enable in kernel config:

  - CONFIG_PSTORE=y

  - CONFIG_PSTORE_RAM=y

- Add bootargs:

pstore.backend=ramoops ramoops.mem_address=0x100000 ramoops.mem_size=0x40000

After a crash → reboot → logs are available at:

ls /sys/fs/pstore/

- Example files:

- console-ramoops → console logs before crash.

- dmesg-ramoops → kernel logs before crash.

- View logs:

cat /sys/fs/pstore/console-ramoops

Explanation for Presentation:

- dmesg → temporary logs (lost after reboot).

- journalctl → persistent runtime logs (stored on disk).

- pstore → crash logs (survive reboot, useful for debugging kernel panics).

# 🔹 Final Summary (Slide/Closing Words

- Step 1: Use of_property_read_* APIs to read DTS properties and verify them in dmesg.

- Step 2: Use journalctl for analyzing runtime and persistent logs.

- Step 3: Use pstore to capture crash logs that survive reboot.

Together, these give a complete debugging framework for embedded Linux BSP development.
1. What happens inside a BSP during system boot-up?

During boot-up, the BSP initializes critical hardware components like CPU, clocks, memory controllers, and peripheral interfaces. It sets up the bootloader environment and ensures the kernel receives proper hardware mappings. This process guarantees a seamless handoff from hardware startup to OS kernel initialization.

### 2. How does BSP help in porting Linux to new hardware?

When bringing Linux to a new board, BSP acts as a bridge between the kernel and the board’s specific hardware. You only need to modify BSP components like the device tree, startup code, and board-specific drivers. The kernel itself remains untouched, enabling rapid adaptation across various hardware platforms.

### 3. Why do we modify .dts and .dtsi files in BSP customization?

The .dts and .dtsi files define hardware configurations for a board. When you extend BSP for custom hardware, you add or modify nodes describing GPIOs, I2C devices, memory maps, and interrupt lines. The kernel then reads these values dynamically at runtime to initialize peripherals correctly without hardcoding addresses.

### 4. How can you verify whether the kernel has read your DTS values correctly?

After loading your module, use dmesg | grep <driver_name> or journalctl -k to check printed logs from of_property_read_*() calls. If properties like IRQ numbers or labels appear correctly, it means the kernel successfully parsed your DTS entries. This step validates correct integration between BSP and driver.

### 5. What is the role of of_property_read_ APIs in BSP development?*

Functions like of_property_read_u32 and of_property_read_string extract property values from the Device Tree node at runtime. Instead of hardcoding register addresses or IRQs, the driver dynamically reads them through these APIs. This makes drivers portable and adaptable to multiple boards without code changes.

### 6. Why is journalctl preferred over dmesg for log analysis in BSP?

While dmesg shows only the current kernel log buffer, journalctl stores logs persistently using systemd’s journal. Even after reboots, you can retrieve past kernel or driver logs for debugging. It’s especially useful in BSP-level debugging where you analyze boot and runtime logs across system restarts.

### 7. What is pstore, and how does it enhance BSP debugging?

pstore (Persistent Storage) stores kernel logs from crashes or panics into a reserved memory region or flash partition. After reboot, these logs remain accessible under /sys/fs/pstore/. It’s an essential BSP feature for post-crash diagnostics, allowing recovery of kernel panic messages that would otherwise be lost.

### 8. How do you enable pstore in the kernel configuration?

You enable it by setting CONFIG_PSTORE=y and CONFIG_PSTORE_RAM=y in kernel config, then specifying boot parameters like pstore.backend=ramoops and ramoops.mem_address=<addr>. After reboot, crash logs can be read from /sys/fs/pstore/. This helps BSP developers capture and analyze crashes systematically.

### 9. What common issues occur when extending BSP for new hardware?

Common issues include incorrect memory mapping, wrong interrupt numbers, or missing compatible strings in DTS. These cause drivers to fail during probe. Debugging involves cross-verifying DTS entries, enabling debug prints, and ensuring hardware initialization sequences match the board schematic and SoC documentation.

### 10. How do you ensure BSP stability after customization?

After integrating new peripherals or modifying the device tree, you perform stress testing and continuous logging using journalctl and pstore. Each driver is validated for correct initialization and removal without memory leaks or race conditions. A stable BSP ensures reliable communication between the kernel and hardware.

### 11. What is the difference between BSP and device drivers?

A BSP initializes and abstracts all hardware-level details to bring up the system, while a driver handles a specific device’s operation. BSP is board-specific, dealing with bootloader, memory, and SoC-level setup. Drivers, on the other hand, are reusable across boards once BSP provides correct hardware configuration.

### 12. Why is BSP testing critical before releasing an embedded product?

Any instability in BSP affects the whole system, from kernel boot to application execution. Thorough BSP testing ensures proper clock, power, and peripheral configuration. It also validates memory timing and device initialization, preventing system hangs or boot failures during field deployment.

### 13. How do BSP developers use dmesg and journalctl together?

During live debugging, dmesg helps observe real-time driver logs, while journalctl provides persistent history across reboots. Developers combine both to correlate runtime events with past crash or boot logs. This dual logging strategy provides a full timeline for diagnosing BSP-level hardware and driver issues.

### 14. How do BSP and bootloader interact during startup?

The bootloader (like U-Boot) initializes low-level hardware and loads the kernel image and device tree into memory. The BSP then takes over once the kernel starts, initializing board-specific peripherals. Together, they ensure smooth hardware-to-kernel transition and successful operating system boot-up.

### 15. What is the impact of BSP optimization on product performance?

A well-optimized BSP improves system boot time, reduces power consumption, and enhances peripheral efficiency. Optimizations like correct clock settings, memory tuning, and minimal driver overhead can significantly improve product stability. In real-world embedded systems, BSP quality directly influences overall device reliability and speed.