Watchdog Timer Driver Implementation on Raspberry Pi 4:

This project focuses on designing and implementing a custom Linux kernel driver for the Raspberry Pi’s built-in hardware watchdog timer, without using any extra external components.
The idea behind this project is to understand how the watchdog subsystem in Linux works and how the kernel interacts directly with hardware registers to maintain system reliability.

### What is a Watchdog Timer?

A watchdog timer is a special hardware timer used to detect and recover from system hangs or software malfunctions.
It continuously counts down from a preset value, and if the software doesn’t “ping” or “reset” it within that time, the watchdog assumes the system is stuck — and automatically reboots the device.

In real-world embedded systems, watchdogs are crucial for safety and reliability — for example, in industrial controllers, automotive ECUs, routers, and IoT devices — where unattended operation must be maintained.

2️ Introducing “Kicking” the Watchdog
After understanding the purpose of the WDT, the next key concept is ‘kicking’ the watchdog, also known as feeding or servicing the timer.

- Definition: Kicking the watchdog means sending a periodic signal from the software to the WDT to indicate that the system is operating correctly.

- Purpose: It prevents the WDT from erroneously assuming that the system is hung, thus avoiding an unnecessary reset.

- Mechanism:

  - The WDT counts down continuously from its preset timeout value.

  - The software must execute a kick command (often via writing to a specific register or using a driver API) before the counter expires.

  - If the kick is missed due to software malfunction (e.g., infinite loop, deadlock, or hardware fault), the WDT reaches zero and triggers a system reset.

### How the Watchdog Works

- The system (CPU/software) starts the watchdog timer and sets a timeout value (say, 5 seconds).

- The WDT begins counting down internally.

- The main software must periodically reset (“kick”, “feed”, or “pet”) the watchdog before the timer expires.

- If the software fails to reset the WDT in time (for example, it hangs in an infinite loop),
➜ The timer expires and triggers a system reset (or interrupt).

- After reset, the system restarts cleanly and continues normal operation.

### Why a Watchdog Timer is Needed

Watchdog timers are essential in embedded systems because these devices often run unattended for long periods (e.g., in cars, routers, or medical devices).
If the system hangs, there is no one to press the reset button — that’s where the WDT saves the day.

#### Reasons We Need a Watchdog:

- Automatic Recovery from Software Failures

  - If a program crashes or gets stuck, WDT resets the system automatically.

- Increases Reliability

  - Ensures continuous operation without human intervention.

- Prevents System Lockup

  - Detects deadlocks, infinite loops, or missed interrupts.

- Safety in Critical Applications

  - In automotive, aerospace, or medical devices, system failure can be dangerous — WDT ensures safe recovery.

- Remote/Unattended Devices

  - IoT devices, routers, or industrial controllers deployed in remote areas can auto-recover without manual reset.

## Types of Watchdog Timers

There are two main types of watchdog timers used in embedded systems:
1️Internal Watchdog Timer
2️ External Watchdog Timer

### 1. Internal Watchdog Timer

Explanation:

- This type of watchdog timer is built inside the microcontroller itself.

- It is controlled through special registers in the MCU.

- The software can enable it, set the timeout period, and “feed” it directly through code.

- If the program fails to feed it within the set time, the MCU resets automatically.

Example:

- In an ARM Cortex-M3, AVR (ATmega), or STM32, there is a built-in WDT that can be started with just a few lines of code.

Advantages:

- No extra hardware needed.

- Easy to configure.

- Low cost and compact design.

Limitations:

- If the MCU itself gets stuck due to a hardware fault, the internal watchdog might also hang (since it’s inside the same chip).

### 2. External Watchdog Timer

Explanation:

- This type uses a separate hardware chip (IC) connected to the microcontroller, usually through GPIO or I²C/SPI lines.

- The microcontroller sends periodic pulses (or signals) to this external IC.

- If the external watchdog doesn’t receive the pulse in time, it triggers a hardware reset to the MCU.

Example:

- ICs like TPS3823, MAX6369, or DS1232 act as external watchdog timers.

- Commonly used in automotive ECUs, industrial controllers, and medical devices where high reliability is critical.

Advantages:

- More reliable because it works independently from the main MCU.

- Can reset the system even if the internal clock or MCU core fails.

- Offers better fault tolerance.

Limitations:

- Slightly increases cost and circuit complexity.

- Requires extra connections and configuration.

### Comparison Table

| Feature | Internal Watchdog | External Watchdog |
| --- | --- | --- |
| Location | Inside MCU | Separate chip |
| Cost | No extra cost | Adds small cost |
| Complexity | Easy to configure | Slightly complex |
| Reliability | Moderate | Very High |
| Used In | Simple systems | Safety-critical systems |
| Example | ARM, AVR built-in WDT | TPS3823, MAX6369 ICs |

### Example Scenario 5: Smart Home Fan Controller

Imagine you have a smart fan that automatically turns ON or OFF based on room temperature.
The fan is controlled by a microcontroller running a program that:

- Reads the temperature sensor,

- Decides when to turn the fan ON or OFF, and

- Sends updates to a mobile app.

Now, suppose the software hangs because of a bug —
the fan might stay ON forever or never turn ON even if the room gets hot.

That’s where the Watchdog Timer (WDT) helps.

Here’s what happens:

- The watchdog is set to 5 seconds.

- As long as the program is working properly, it feeds the watchdog every few seconds.

- If the program stops feeding it (because it got stuck),
➜ The watchdog timer expires,
➜ It resets the microcontroller,
➜ The system restarts and begins working normally again.

To interact with the hardware watchdog on the Raspberry Pi 4, we need to understand and manipulate three key memory-mapped registers in the Broadcom SoC. These registers are used to configure, start, and ping the watchdog timer.

## Register 1: PM_WDOG — Watchdog Timer Register

### How to Explain:

“The PM_WDOG register is where we set the countdown value for the watchdog timer. The unit of this value is 1 tick = 1/16th of a second. So if we want the watchdog to expire in 10 seconds, we write 10 * 16 = 160 ticks to this register.”

### Analogy:

“Think of it like setting the timer on a microwave. You press 10 seconds, and it counts down. Same thing here — but it’s in ticks.”

### Code Example:

#define PM_WDOG 0x10000024

#define PM_PASSWORD 0x5A000000

writel(PM_PASSWORD | (timeout << 4), PM_WDOG);

## Register 2: PM_RSTC — Reset Control Register

### How to Explain:

“Once we set the timer, we also need to tell the SoC what to do when the timer expires. That’s done using the PM_RSTC register. We write a special bit called WRCFG_FULL_RESET to trigger a full board reset when the watchdog expires.”

Code Example:

#define PM_RSTC 0x1000001C

#define PM_RSTC_WRCFG_FULL_RESET 0x00000020

writel(PM_PASSWORD | PM_RSTC_WRCFG_FULL_RESET, PM_RSTC);

Analogy:

“This register is like the 'reset action' on a smart plug. When the timer runs out, do you want to beep? Flash? Or turn off power? We’re saying: Reset the whole system.”

## Important Concept: PM_PASSWORD

### How to Explain:

“Every write to these watchdog registers must be prefixed with a magic password: 0x5A000000. This acts as a protection mechanism to prevent accidental writes to critical system registers.”

### Example:

#define PM_PASSWORD 0x5A000000

If you don’t include the password, the write is ignored.”

### What is PM_PASSWORD then?

- It’s a magic constant (a special number) that you must include in every write operation to certain hardware watchdog registers on the Raspberry Pi (specifically to the Power Management registers controlling the watchdog).

- Think of it like a password or a key required by the hardware to accept the write — a safety feature to prevent accidental or spurious writes that could cause system instability.

### How it works in practice:

- The hardware expects that any write to watchdog-related registers includes this password in the upper bits of the 32-bit write value.

- If the password bits are missing or incorrect, the write is ignored — the hardware won’t accept or process it.

## How to "Ping" or "Kick" the Watchdog

### How to Explain:

“To prevent the watchdog from resetting the system, we must periodically rewrite the timeout — this is known as kicking or pinging the watchdog.”

### Code:

writel(PM_PASSWORD | (timeout << 4), PM_WDOG);

In our driver, this goes into the .ping() function of the watchdog_ops structure.”

## How to Stop the Watchdog (Optional)

“Some watchdog implementations don’t allow stopping once started (called nowayout), but if supported, we can clear the countdown and reset config.”

### Example (if supported):

writel(0, PM_WDOG);

writel(PM_PASSWORD | 0, PM_RSTC);

Summary of Register Use in Driver

| Operation | Register | Value |
| --- | --- | --- |
| Set timeout | PM_WDOG | `PM_PASSWORD |
| Configure reset | PM_RSTC | `PM_PASSWORD |
| Ping/kick | PM_WDOG | Same as setting timeout |
| Stop (optional) | PM_WDOG, PM_RSTC | Clear values |

## Final Explanation Style (scripted example):

“So, in summary, the watchdog timer on the Raspberry Pi 4 is controlled through three main registers: PM_WDOG to set the countdown, PM_RSTC to define the reset behavior, and a mandatory PM_PASSWORD prefix to ensure secure access.
Each time we want to start or ping the watchdog, we write the timeout value to PM_WDOG, and configure the reset action using PM_RSTC.
The Linux driver interacts with these registers using memory-mapped I/O and wraps this logic into start, stop, and ping functions that integrate with the Linux watchdog framework.”

## What is ioremap and Why Do We Use It?

### Physical vs Virtual Memory in Linux Kernel

- The hardware registers controlling peripherals like the watchdog timer reside at specific physical memory addresses on the SoC (System on Chip).

- However, the Linux kernel runs in a virtual memory space, and cannot directly access physical addresses.

- To safely access hardware registers, the kernel must map these physical addresses into its virtual address space.

### What Does ioremap Do?

- ioremap() takes a physical address range (like the watchdog's registers) and returns a virtual address pointer that the kernel can use.

- This mapping allows the driver to read/write hardware registers as if they were normal memory.

## Why Do We Write to These Registers in Start / Ping / Stop?

- Start:

  - Set the watchdog timeout (PM_WDOG) to the desired value (e.g., 10 seconds).

  - Set reset mode and enable watchdog reset via PM_RSTC.

- Ping:

  - Refresh (reload) the countdown timer by writing again to PM_WDOG.

  - This prevents the watchdog from expiring.

- Stop:

  - Write zero or disable the reset control (if supported).

  - Not all watchdog hardware supports disabling once started, but your code attempts it by zeroing the reset control.

Perfect — here's a presentation-ready version with detailed, spoken-style sentences that you can read out directly during your presentation. Each section explains one of the four important watchdog registers on the Raspberry Pi 4, with deeper insights and code snippets where needed.

# Raspberry Pi 4 Watchdog — Important Registers Explained for Presentation

### 🔹 1. PM_WDOG – Watchdog Timer Register

Let’s start with the PM_WDOG register, which is the core of the watchdog system.
This register is responsible for storing the watchdog countdown value — essentially the time remaining before the system is reset.

In the Raspberry Pi architecture, this timeout is not set in seconds directly, but rather in ticks, where each tick represents 1⁄16th of a second.
That means if I want a 10-second watchdog timeout, I need to write a value of 10 multiplied by 16, which is 160 ticks.

This value must be written along with a specific password, 0x5A000000, which must be placed in the top 8 bits of the 32-bit value.

Yes — the magic password (0x5A in the top byte) is chosen by the SoC manufacturer (Broadcom) and implemented in hardware. It’s not a software key you can change at runtime — it’s a fixed hardware protection mechanism baked into the PM/reset/watchdog logic of the chip.
The hardware will ignore any write operation that does not include this password, so it is mandatory.

This register is located at memory address 0xFE100024.

Here’s a code example that sets a 10-second watchdog timer:

#define PM_PASSWORD 0x5A000000

#define WDOG_TIMEOUT 10

iowrite32(PM_PASSWORD | (WDOG_TIMEOUT * 16), wdog_base + 0x24);

Writing to this register both starts the watchdog and is also used to ping or reset the timer to prevent a reset.

Broadcom uses 0x5A as a special key to unlock access to critical registers like the watchdog timer.
This value is special because it has a unique binary pattern (01011010) that stands out clearly when seen in memory or register dumps. It was carefully chosen by Broadcom, not randomly.

It doesn’t happen by accident because most random or faulty memory writes won’t produce this exact value in the top byte of a 32-bit register. That makes it very unlikely for the system to accidentally reset or misbehave due to a wrong write.

This helps protect the system from accidental resets or unwanted changes by making sure only intentional and correct writes can control important hardware like the watchdog timer.

### 2. PM_RSTC – Reset Control Register

Next, we have the PM_RSTC register, which stands for Power Management Reset Control.
This register is used to define how the system should be reset when the watchdog timer expires.

While PM_WDOG sets the timeout, PM_RSTC sets the action to take when that timeout expires.

The most important field here is the WRCFG bits, specifically bits 4 and 5.
Setting these bits to the value 0x20 tells the system to perform a full system reset when the watchdog expires.
Like before, we also need to include the 0x5A password in the top 8 bits.

This register is located at address 0xFE10001C.

To enable a full system reset, we would write the following:

#define PM_RSTC_WRCFG_FULL_RESET 0x00000020

iowrite32(PM_PASSWORD | PM_RSTC_WRCFG_FULL_RESET, wdog_base + 0x1C);

To stop or disable the watchdog reset, we can clear the reset type bits like this:

#define PM_RSTC_WRCFG_NONE 0x00000000

iowrite32(PM_PASSWORD | PM_RSTC_WRCFG_NONE, wdog_base + 0x1C);

So in summary, PM_RSTC defines whether the system should perform a full reset or no reset when the watchdog expires, and it must be configured correctly for the watchdog to function as intended.

### 3. PM_RSTS – Reset Status Register

The third register is PM_RSTS, which stands for Reset Status.
This is a read-only register that provides information about why the last system reset occurred.

This is incredibly useful when debugging — for example, you can use it to verify if the system reset was caused by the watchdog, or due to some other reason like a power-on reset or a software reboot command.

This register is located at address 0xFE100020.

The bit we are most interested in is bit 4, which is set when the system is reset due to the watchdog.

Here’s an example of how to check this:

u32 status = ioread32(wdog_base + 0x20);

if (status & (1 << 4)) {

pr_info("Last reset was due to watchdog\n");

}

So in a real-world scenario, after the system boots up, your driver or program can read this register to log or confirm that a watchdog reset actually occurred, which helps prove that the watchdog is working correctly.

### 4. PM_WDOG (Used Again for Keep-Alive / Ping)

Finally, we come back to PM_WDOG, which is used again to “ping” or “kick” the watchdog.
After the watchdog has been started, you need to periodically reset or “refresh” the countdown timer, or else the system will reset once the timer expires.

This is done by writing the same timeout value to the same register again — and again, we must include the password.

So every few seconds, your driver or user-space application should write the following line:

iowrite32(PM_PASSWORD | (WDOG_TIMEOUT * 16), wdog_base + 0x24);

This resets the internal countdown, and if you forget to ping the watchdog, the system will reset.

So this register acts both as a starter and a lifeline for keeping the watchdog alive.

## Putting It All Together – Watchdog Lifecycle

Let’s briefly look at the typical sequence of operations when using the watchdog timer:

- First, you start the watchdog by writing the timeout value to PM_WDOG and enabling reset in PM_RSTC.

- Next, in your driver or user-space application, you must periodically ping the watchdog by writing to PM_WDOG again to prevent reset.

- If needed, you can disable or stop the watchdog by clearing the reset type in PM_RSTC.

- Finally, if the system reboots, you can check PM_RSTS to confirm whether the watchdog was the cause.

## Summary of the Four Registers

| Register | Address | Purpose |
| --- | --- | --- |
| PM_WDOG | 0xFE100024 | Set or refresh watchdog timeout |
| PM_RSTC | 0xFE10001C | Configure system reset behavior |
| PM_RSTS | 0xFE100020 | Read status of last system reset |
| PM_WDOG | 0xFE100024 | Used again to ping the watchdog |

## Final Thoughts

To summarize:
The PM_WDOG register handles the countdown.
The PM_RSTC register determines what kind of reset happens.
The PM_RSTS register helps you confirm what caused the last reset.
And pinging the PM_WDOG register repeatedly is the key to preventing unexpected system resets.

These four registers work together to implement a reliable, low-level watchdog system on the Raspberry Pi 4, giving you precise control over system health and recovery.