Good morning everyone. My name is Chittajallu Swarna Latha, and today I’m going to present my major project titled:

Secure Real-Time Temperature Monitoring System with Kernel-Level Access Control and Remote Alerts

🟦 Secure
"In our project, we added a password mechanism, so only authorized users can change the temperature threshold. If someone enters the wrong password three times, the system locks itself automatically."

🟦 Real-Time
"Our system keeps reading the temperature continuously using a sensor. If the temperature goes above the threshold, it reacts immediately without any delay."

🟦 Kernel-Level Access Control
"We didn’t just write user programs — all security and control logic like password checking and monitoring is written inside the Linux kernel as a device driver using IOCTL and kernel threads."

🟦 Remote Alerts
"If the temperature crosses the safe limit, the Raspberry Pi sends a socket message to another system, which plays an audio alert to notify people nearby instantly."

This project was developed using embedded Linux concepts like kernel modules, IOCTLs, kthreads, waitqueues, and I2C protocols, combining both hardware and software in a real-time monitoring system.”

### Problem Statement

“In many industrial and storage environments like chemical storage units, medical vaccine fridges, or food warehouses, maintaining the correct temperature is very critical. A sudden increase can lead to chemical spoilage, medicine becoming ineffective, or food rotting.

So there is a strong need for:

- A real-time temperature monitoring system.

- Only authorized people should be able to configure thresholds.

- If temperature exceeds, the system should alert immediately.

Hence, our goal is to build a secure, real-time monitoring system where only verified users can change the threshold and the system gives alerts if the temperature exceeds the limit.”

### Real-Time Use Case Examples

“This system can be used in:

- Cold storage warehouses to ensure food is stored at safe temperatures.

- Vaccine or medicine storage where temperature must not exceed limits.

- Chemical labs where overheat may cause explosions or spoilage.

- Even server rooms where overheating needs to be detected early.

In all these scenarios, our project can ensure only authorized personnel can change temperature limits, and alerts are sent automatically in case of risk.”

his project is titled: "Secure Kernel-Level Temperature Monitoring System using Raspberry Pi, UART, OLED, and Socket-Based Audio Alerts".

- It comprises three main devices: x86_1 for secure input, Raspberry Pi for kernel-based monitoring, and x86_2 for audio alert generation.

- The user enters a password and temperature threshold on x86_1, which sends data to the Raspberry Pi over UART communication.

- The x86_1 kernel driver handles IOCTL-based password/threshold storage and UART transmission to the Pi.

- On the Raspberry Pi side, the user-space program reads UART data and sends it to the kernel module via IOCTLs.

- The Raspberry Pi kernel module stores the password and requires verification before enabling sensor monitoring.

- If the user enters the correct password, monitoring begins; otherwise, 3 wrong attempts lock the system and trigger an alert.

- The DS18B20 sensor is used to read ambient temperature every 2 seconds in a kernel thread.

- Readings are parsed from /sys/bus/w1/devices/.../w1_slave in the kernel using filp_open and kernel_read.

- If the measured temperature exceeds the configured threshold, the Pi kernel sends an alert message via TCP socket to x86_2.

- The x86_2 system, which runs a socket server, plays an audio file using aplay as soon as it receives this message.

- The Raspberry Pi also shows real-time temperature and threshold on an OLED display by invoking a user-space display program via call_usermodehelper.

- If 3 password failures occur, a separate "LOCKED" message is sent to the OLED and alert audio is sent to x86_2.

- All critical kernel variables (password, threshold, flags) are protected using mutex locks.

- Monitoring is delayed until password verification by using a wait queue, ensuring secure access flow.

- Communication between kernel and user space is achieved using IOCTLs, enabling precise control over password and threshold operations.

- The entire system is modular and follows good embedded design practices, separating user space, driver logic, and communication.

- The project demonstrates integration of UART, socket, and I2C protocols within an embedded Linux environment.

- It also offers insights into real-world kernel module development, concurrency control, and inter-device signaling.

- Overall, this project delivers a robust, secure, real-time, and multi-device embedded monitoring solution suitable for mission-critical use cases.

This project combines both hardware and software using Embedded Linux.
We used a DS18B20 temperature sensor to read the temperature and an OLED display to show it.
The system runs on a Raspberry Pi, which receives settings from an x86 system through UART communication.
To ensure security, only users who enter the correct password can set the threshold.
After the password is verified, the Raspberry Pi starts monitoring the temperature.
If the temperature exceeds the limit, it sends an alert to another x86 system using socket programming.
That system then plays an audio alert to inform nearby people.
Inside the Linux kernel, we used IOCTL, kernel threads, wait queues, and usermode helper to make it work.
The entire flow runs at the kernel level, making it faster and more reliable.
This project is ideal for places where temperature control and security are very important.

### 1. Why did you use IOCTL instead of write/read syscalls for configuration?

IOCTL allows fine-grained control and passing structured commands like setting password and threshold, which are not stream-based.
It ensures clear distinction between different operations, unlike ambiguous write() calls.
Also, IOCTL enables direct mapping to specific kernel actions, improving modularity and maintainability.With IOCTL, we can clearly define what each command does, so the driver code becomes easier to manage and understand.

### 2. What is the purpose of using a kernel thread in your Raspberry Pi driver?

The kernel thread (kthread_run) continuously monitors temperature every 2 seconds.
It avoids blocking user-space and provides a dedicated monitoring loop inside the kernel.
This enables real-time decisions like triggering alerts or updating OLED without delays.

### 3. Why is a wait queue used before starting temperature monitoring?

The wait queue ensures monitoring only starts after password verification, adding a security layer.
wait_event_interruptible blocks the thread until start_monitoring is true.
It avoids premature sensor access and synchronizes user-kernel interaction efficiently.

### 4. How do you protect shared resources in your kernel driver?

We use a mutex (rpi_mutex) to guard shared variables like password, threshold, and flags.
This ensures safe access in concurrent ioctl calls and kthread execution.
It prevents race conditions and maintains driver stability under concurrent operations.

### 5. Why did you use sockets for communication between Raspberry Pi and x86_2?

Sockets provide flexible inter-device communication, even over a LAN.
We use TCP sockets for reliable delivery of alerts (like audio triggers).
It's scalable and can be extended to multiple clients or cloud alerts in future.

### 6. What is the role of call_usermodehelper() in your driver?

This function allows kernel-space code to run a user-space program, here used to update OLED display.
We use it to call /usr/bin/oled_display with temp and threshold as arguments.
This bridges kernel-user graphics interaction without writing I2C code inside kernel.

### 7. How is the DS18B20 sensor accessed from the kernel?

Instead of using I2C directly, we read temperature from its sysfs file under /sys/bus/w1/.../w1_slave.
We open the file in kernel using filp_open, then parse "t=" to extract temperature.
This method simplifies integration and works well with one-wire kernel drivers.

### 8. How did you implement password locking after 3 wrong attempts?

We maintain a failed_attempts counter, and after 3 wrong verifications, we call
call_usermodehelper to show “LOCKED” on OLED and send an audio alert over socket.
This ensures account lockout logic, similar to real authentication systems.

### 9. What is the format of data sent over UART from x86_1?

The x86_1 driver sends a string like PASS:abc123 THRESH:45\n to /dev/ttyUSB0.
This format is parsed by Raspberry Pi user space using sscanf.
Clear formatting ensures reliable data extraction on the receiving end.

### 10. Why do you open /dev/ttyUSB0 in kernel space on x86_1?

The x86_1 driver handles UART transmission, so it uses filp_open("/dev/ttyUSB0").
This allows sending password and threshold directly from kernel to UART.
It simplifies the system by letting the driver handle low-level communication.

### 11. How do you handle invalid UART messages on the Raspberry Pi side?

The user program checks the UART string format using sscanf.
If the pattern doesn't match PASS:<pass> THRESH:<val>, an error is shown.
This validates incoming data and avoids garbage being passed to the kernel.

### 12. What happens if the threshold is sent before password verification?

In that case, the kernel returns -EPERM from IOCTL and shows a warning.
This enforces a strict sequence — password must be verified first.
It prevents unauthorized users from triggering monitoring.

### 13. What is the purpose of the OLED display in your system?

OLED provides a real-time visual output of current temperature and threshold.
It also shows system status like “LOCKED” or “NORMAL”.
This improves usability and user feedback during operation.

### 14. How is audio played on the x86_2 system when alerts are received?

The x86_2 user program listens on port 8080 using a TCP server.
When it receives a string like alert_audio1.wav, it calls system("aplay <file>").
Thus, audio is played automatically based on alert type.

### 15. What are the risks of accessing GPIO or I2C directly from kernel?

Direct access without synchronization or drivers can lead to bus contention, crashes, or instability.
Instead, we move display logic to user space and call it from kernel using usermode helper.
This separates concerns and keeps kernel code safe and simple.

### 16. Can this system work in a commercial/production setting?

Yes, with a few improvements like robust password encryption, better display integration, and error recovery,
it can be scaled into a production monitoring system.
It already uses real interfaces, kernel security, and socket-based notifications.

### 17. What kernel structures or concepts did you learn through this project?

I worked deeply with file_operations, IOCTL handling, wait queues, kernel threads, mutexes, and
kernel-to-user interaction using call_usermodehelper.
Also learned about socket APIs and device file creation using class_create and device_create.

### 18. What makes this system secure compared to basic monitoring?

Unlike basic sensor readers, we implement password-based access control and restrict critical operations.
Wrong access is logged, alerts are raised, and the system locks after repeated failures.
This adds authentication, authorization, and alerting, making it a secure embedded system.

### 19. How can this system be extended in future?

We can add multiple sensor support, SMS/email alerts, web interface for remote control, and
cloud integration with MQTT or HTTPS.
Also, adding encrypted communication and logging improves security and traceability.

### 20. Why did you split your project into 5 separate code files/modules?

Each part of the project has a dedicated responsibility:
x86_1 for data entry, Raspberry Pi for secure monitoring, and x86_2 for alert playback.
This modularity makes debugging easier and matches real-world multi-device system designs.

## PART 1: High-Level Overview (rpi_user)

The rpi_user.c program acts as the user-space interface for the Raspberry Pi’s secure temperature monitoring system. Its primary role is to communicate with the Raspberry Pi kernel driver (/dev/rpi_secure) via IOCTL calls and send password and temperature threshold values after optionally receiving them from the x86 system via UART. It also provides options for manual password verification and dynamic threshold configuration.

This user-space code is crucial because kernel modules cannot directly interact with users, so a middle-layer like this is needed to accept inputs and issue commands. It allows runtime configuration without needing to reload or recompile kernel code.

Here’s what the code enables:

- Accepting password/threshold from UART and forwarding to the driver.

- Allowing manual password entry and verifying against kernel-stored password.

- Modifying temperature threshold from userspace without a reboot.

- Sending command/control data using ioctl to the kernel character device /dev/rpi_secure.

This file bridges hardware inputs (like UART data) and user inputs (keyboard/manual entry) to the secure kernel logic.

This design is modular, extendable, and secure. For example, in a real-world deployment, this userspace app could run as a daemon, collecting config data from a secure server, web interface, or file. It also simplifies testing, allowing developers to simulate password attempts or update thresholds without modifying kernel code.

Additional possibilities:

- GUI integration to show OLED temperature stats and system status.

- Logging failed password attempts and alerts to a syslog or file.

- Accepting multiple user roles with varying permissions.

- Integrating a watchdog or auto-reset logic based on timeout or errors.

## Overview: What rpi_driver.c Does

The rpi_driver.c file is a Linux kernel module specifically designed for Raspberry Pi. It functions as a secure monitoring kernel driver that receives configuration data (password and threshold) via IOCTL, monitors the DS18B20 temperature sensor, and displays the temperature and system status ("Locked"/"Normal") on an OLED display using a userspace helper (file.c). It also sends alert messages to an x86_2 system via socket if the temperature crosses a user-defined threshold.

This driver is central to the security and monitoring logic:

- Accepts password and threshold from user-space via IOCTL

- Waits until correct password is received

- Spawns a kernel thread to monitor temperature

- Uses tasklets or workqueues to handle OLED display

- Sends alert via socket on threshold violation

- Locks the system after 3 failed password attempts

- Communicates securely via character device and kernel interfaces

## ✅ Importance of This Code

- Demonstrates secure kernel-user space communication using IOCTL

- Integrates sensor (DS18B20) and I2C OLED display

- Uses kthread, waitqueues, tasklets/workqueues, which are core Linux kernel concepts

- Includes password authentication and security logic

- Adds alerting mechanism via socket, making it useful for real-world critical monitoring

- Teaches how to call a user-mode program (file.c) from kernel space using call_usermodehelper()

### Complete Step-by-Step Flow of rpi_driver.c (50+ Big Steps)

- When the kernel module is inserted, the function rpi_init() is called automatically because it is marked with module_init().

- Inside rpi_init(), the alloc_chrdev_region() function dynamically allocates a major and minor number for the character device.

- Then, class_create() is called to create a device class in /sys/class, helping udev to manage the device.

- The device_create() function creates the device node /dev/rpi_secure, which userspace apps (like rpi_user) can access.

- cdev_init() is used to link the device operations (open, read, write, ioctl) to the file operations structure.

- Then cdev_add() adds the device to the kernel, finalizing the registration of our secure kernel module.

- Initialization is now complete, and the module is ready to accept operations from user space.

- When the user runs rpi_user.c and opens the device file (/dev/rpi_secure), the kernel calls the rpi_open() function.

- rpi_open() prints a debug message: "Device opened", and allows the session to proceed. No restrictions are placed here.

- If the user closes the device, rpi_release() gets triggered and shows "Device closed".

- When the user chooses Option 1 from rpi_user (UART read), it writes to the device. So, rpi_write() is triggered.

- In rpi_write(), the input string is copied from user space into kernel space using copy_from_user().

- The string is expected in the format "PASS:<password> THRESH:<value>".

- The function sscanf() is used to extract both the password and threshold.

- The password is stored in stored_password, and threshold is stored in threshold_temp.

- A log message confirms both values have been received and stored securely.

- When the user chooses Option 2, ioctl() is triggered with VERIFY_PASSWORD.

- Inside rpi_ioctl(), the VERIFY_PASSWORD case is handled first.

- The password is received from user space using copy_from_user() into a buffer.

- Then, strncmp() is used to compare user input with the stored password.

- If they match, password_verified = 1 is set, and the waitqueue wait_event_interruptible() can wake up.

- If the password is incorrect, attempts are incremented.

- After 3 wrong attempts, locked = 1 is set, preventing further sensor operations.

- A status message “System locked” is written into /tmp/status.txt for the OLED display program to read.

- Additionally, a socket alert is sent to x86_2_user.c using sock_send_alert(), which uses sock_create, connect, and kernel_sendmsg.

- When the password is correct, the system starts the kernel thread for monitoring by calling kthread_run(sensor_thread,...).

- sensor_thread() is a kernel thread function that runs in background to continuously monitor temperature.

- It starts with a wait_event_interruptible() on the wait queue until password_verified becomes 1.

- After password verification, the thread loops forever unless kthread_should_stop() is called.

- Inside the loop, it opens /sys/bus/w1/devices/.../w1_slave file using filp_open().

- It reads the sensor output using kernel_read() and extracts the temperature value.

- This value is converted from string to integer and divided by 1000 to get the Celsius value.

- The current temperature and threshold are stored in cur_temp and threshold_temp.

- Then, call_usermodehelper() is used to call the file1.c program, passing temperature and threshold as arguments.

- This helper program displays the data on an OLED screen over I2C.

- Next, the kernel checks if the system is locked (locked == 1).

- If yes, it again sends a socket alert to x86_2_user.c and writes a “Locked” status for OLED.

- If not locked and the temperature exceeds threshold, a socket alert is also sent to play an alarm.

- The loop then sleeps for 5 seconds using ssleep(5) before the next sensor check.

- All this while, logs are being printed for debugging and demonstration purposes.

- The user can also send a new threshold using IOCTL SET_THRESHOLD, which updates threshold_temp.

- If password was not verified, the ioctl call for SET_THRESHOLD will fail.

- The read function (rpi_read()) is defined but not implemented (optional extension for future).

- Throughout the code, mutex_lock() and mutex_unlock() are used to protect shared data.

- If module is removed using rmmod, rpi_exit() is called due to module_exit().

- device_destroy() and class_destroy() clean up the /dev and /sys/class entries.

- cdev_del() and unregister_chrdev_region() release the allocated major and minor numbers.

- The sensor thread is stopped using kthread_stop() to safely end background execution.

- All memory, sockets, threads, and device structures are cleaned up.

- The system is now cleanly removed, and all secure operations are concluded.

- This kernel module integrates security (password lock), hardware interfacing (DS18B20 & OLED), multi-system communication (sockets), and user interaction (ioctl), all in one compact system

### #define DEVICE_NAME "rpi_secure"

- This macro defines the name of the character device that will be created in /dev/.

- When the driver registers using register_chrdev, this name is used for the entry in /dev/rpi_secure.

- It allows the user-space application to open and interact with this device file to communicate with the kernel module.

### 🔹 #define CLASS_NAME "rpi_secure_class"

- Defines the name of the device class used when creating the /dev/rpi_secure file.

- This class helps group the device under /sys/class/ and supports udev rules for auto device creation.

- It plays a role in abstracting hardware devices and linking them to their device nodes.

### 🔹 #define IOCTL_SEND_PASSWORD _IOW('x', 1, char *)

- This is a custom IOCTL command to send the password string from user space to the kernel.

- _IOW indicates a "Write" operation from user space to the kernel using a char * type.

- 'x' is a unique identifier (magic number) to distinguish your driver from others; 1 is the command number.

### 🔹 #define IOCTL_VERIFY_PASSWORD _IOW('x', 2, char *)

- This is another IOCTL command, also writing from user to kernel, but this time to verify the password.

- It takes a password from the user space and compares it with the stored password in the kernel.

- If the password matches, it enables system functionality like starting monitoring.

### 🔹 #define IOCTL_SEND_THRESHOLD _IOW('x', 3, int)

- IOCTL command to send the temperature threshold from the user-space program to the kernel module.

- Once the threshold is set, the kernel uses it to determine when to raise a temperature alert.

### 🔹 #define TEMP_SENSOR_PATH "/sys/bus/w1/devices/28-0316721be1ff/w1_slave"

- This is the absolute path to the DS18B20 sensor’s data file in the Linux sysfs filesystem.

- Your kernel thread opens and reads this file periodically to obtain the current temperature.

- The exact device ID (28-0316721be1ff) will differ depending on your sensor.

### 🔹 #define TEMP_CHECK_INTERVAL_MS 2000

- Defines the interval at which the temperature should be polled (every 2000 milliseconds or 2 seconds).

- This constant is used inside the monitoring kernel thread (monitor_fn) using msleep() for periodic checking.

- Controls how frequently alerts or OLED updates can happen.

### 🔹 #define ALERT_THRESHOLD_AUDIO "alert_audio2.wav\n"

- Represents the filename of the audio file that should be played when the temperature exceeds the threshold.

- This string is sent over a socket to the x86-2 system, which is responsible for playing the audio.

- The newline (\n) is optional unless needed for parsing; remove it if not required.

### 🔹 #define ALERT_PASSWORD_FAIL_AUDIO "alert_audio1.wav\n"

- This is the name of the audio alert that should be triggered when there are three failed password attempts.

- It's also sent via socket to the second x86 system, where an audio playback application handles it.

### 🔹 static int major;

- Stores the major number assigned by the kernel for this character device.

- Used to uniquely identify the driver during registration and when creating the device node /dev/rpi_secure.

### 🔹 static struct class* rpi_class = NULL;

- A pointer to the device class used when creating the device node in /dev.

- Created using class_create() and is used along with device_create() for automatic node creation in /dev.

### 🔹 static struct device* rpi_device = NULL;

- Pointer to the device structure representing the actual /dev/rpi_secure device.

- Created with device_create(), and destroyed during cleanup.

- Links the class and device driver with the device node.

### 🔹 static char stored_password[32] = "";

- Stores the password received from user space using IOCTL_SEND_PASSWORD.

- Used internally for validating password attempts through the IOCTL_VERIFY_PASSWORD interface.

- Has a maximum size of 31 characters + null terminator.

### 🔹 static int stored_threshold = 0;

- Stores the temperature threshold received from the user via IOCTL_SEND_THRESHOLD.

- The monitor_fn checks current temperature against this value to determine whether to trigger an alert.

### 🔹 static bool password_verified = false;

- A flag to indicate whether the correct password has been entered.

- Until this is set to true, the kernel module won't allow temperature monitoring or threshold setting.

### 🔹 static int failed_attempts = 0;

- Keeps track of how many times a wrong password has been entered.

- After 3 failed attempts, the system sends an alert and stops accepting further input.

### 🔹 static struct socket *alert_sock = NULL;

- Kernel-space socket used to send messages (like "alert_audio2.wav") to the x86-2 system.

- Created using sock_create_kern() and released on exit.

- Enables inter-system communication between Raspberry Pi and external x86 device.

### 🔹 static struct task_struct *monitor_thread = NULL;

- Points to the kernel thread responsible for periodic temperature monitoring (monitor_fn).

- Created using kthread_run() and must be explicitly stopped during module unload using kthread_stop().

### 🔹 static DECLARE_WAIT_QUEUE_HEAD(start_monitor_wait);

- Declares a wait queue named start_monitor_wait.

- This queue is used to delay the monitoring thread until password verification is successful (start_monitoring = true).

- The thread sleeps on this queue using wait_event_interruptible() until it is woken up.

### 🔹 static bool start_monitoring = false;

- Used to control the start of the temperature monitoring thread.

- Set to true after password is successfully verified.

- Prevents the monitor thread from accessing sensor/OLED too early.

### 🔹 static DEFINE_MUTEX(rpi_mutex);

- Declares and initializes a mutex for synchronizing access to shared data like password, threshold, and flags.

- Prevents race conditions when user space writes or verifies data concurrently.

- Use mutex_lock() and mutex_unlock() around critical sections.