U-Boot is a key component in the boot sequence of many systems because it initializes the hardware, configures the environment, and loads the operating system, such as Linux or other embedded systems.

### Key Functions of U-Boot

1. Hardware Initialization: U-Boot performs the initial configuration of the processor, memory, I/O devices, and other essential peripherals. This includes setting up clocks, communication buses, and memory controllers.

2. Loading the Operating System: After initialization, U-Boot loads the operating system kernel, such as Linux, and transfers it into memory. It can also load and configure other components, such as the device tree, which describes the hardware of the system.

3. Support for Storage Media: U-Boot supports a variety of storage devices, including NAND flash, microSD, hard drives, and even network devices (via PXE), enabling the system to be loaded from different data sources.

4. Command-Line Interface (CLI): U-Boot features a powerful command-line interface that allows developers to interact with the system during the boot process. This is helpful for debugging and configuring parameters before the operating system is loaded.

5. Script Support: U-Boot allows the execution of automated scripts that can perform tasks such as network configuration, firmware updates, and other boot-related procedures. This is especially useful in systems that require dynamic and automated configuration.

## What's Happening Internally? (Memory Perspective)

### 1. 🧷 BROM (Boot ROM)

- Stored in ROM inside the SoC.

- Very small, non-modifiable, fixed by the chip manufacturer.

- Purpose:

  - Initializes minimal hardware (e.g., clock, watchdog, internal SRAM).

  - Loads the SPL from storage (e.g., SD, eMMC, SPI NOR).

- Memory Involved:

  - Executes from on-chip ROM.

  - Uses small on-chip SRAM for its own stack.

### 2. 🧱 SPL (Secondary Program Loader)

- Runs from on-chip SRAM, typically:

  - 32KB–128KB of Internal RAM (IRAM or TCM).

- Key Responsibilities:

  - Initialize DRAM (external RAM).

  - Set up basic peripherals (e.g., UART, NAND, MMC).

  - Load U-Boot proper from storage into DRAM.

- Memory Usage:

  - Code and data reside in on-chip SRAM.

  - Extremely size-constrained.

    - Often built with CONFIG_SPL_ options to strip out unnecessary features (e.g., shell, network).

  - Typical SPL size: ~20–40KB.

- Think of SPL as the "bare-minimum bootloader".

### 3. 🧠 DRAM Initialization

- This is the most critical job of SPL.

- Before DRAM is initialized, you cannot load or run large binaries (like U-Boot proper or Linux).

- SPL runs RAM initialization routines (DDR PHY training, controller setup).

- Once DRAM is ready:

  - SPL copies U-Boot proper from storage to DRAM.

  - Transfers control to it.

### 4. 🚀 U-Boot Proper

- Runs from external DRAM (DDR).

- Fully featured bootloader:

  - Shell

  - Environment variables

  - Networking (TFTP, DHCP)

  - Filesystems (FAT, ext4)

  - USB, MMC, SPI, I2C drivers

  - Scripts (e.g., boot.scr)

- Memory View:

  - Code, BSS, and heap are all in DRAM.

  - Uses memory mapping to access peripherals.

  - Can allocate buffers, decompress images, and load large binaries (like kernel/initrd).

- Typical Size:

  - ~300KB to 700KB (can be larger if more features are included).

### 5. 🐧 Kernel + DTB + Initrd

- U-Boot proper loads:

  - zImage/Image: Linux Kernel binary.

  - DTB: Device Tree Blob.

  - Initrd (optional): Initial RAM disk.

- Memory Layout Example (in DRAM):

0x80000000 - DRAM base

↓

0x80008000 - Kernel load address (zImage/Image)

↓

0x81F00000 - Initrd (if used)

↓

0x82000000 - Device Tree Blob

↓

0x83F00000 - U-Boot environment, heap, malloc pool

## Summary Table – Memory Usage at Each Stage

| Stage | Runs From | Memory Used | Main Role |
| --- | --- | --- | --- |
| BROM | On-chip ROM | ROM + SRAM | Load SPL |
| SPL | On-chip SRAM | ~32–128KB | Init DRAM, load U-Boot proper |
| U-Boot | DRAM | MBs | Load kernel, init peripherals |
| Kernel | DRAM | MBs | Start OS |

### What is a Bootloader?

A bootloader is a small program that runs right after a device powers on and is responsible for loading and starting the operating system (OS).

Think of it as the bridge between hardware and the OS.

## 🧠 Simple Definition:

A bootloader is the first software that runs after power-on and prepares the system to run the main operating system.

## 🧭 Key Responsibilities:

- Hardware Initialization

  - Set up CPU, clocks, memory (RAM), and peripherals.

  - On some platforms, even initializes DDR RAM (via SPL or TPL).

- Loading the OS

  - Loads the kernel image (e.g., zImage, Image, or vmlinuz) into RAM.

  - Loads the Device Tree Blob (DTB) and optionally an initrd/initramfs.

- Passing Control to OS

  - Sets up boot arguments.

  - Jumps to the OS entry point to hand off control.

- (Optional) Recovery/Debug Options

  - Can provide shell access, networking, flashing tools, etc.

## 🛠️ Bootloader Stages (in embedded systems like U-Boot):

| Stage | Purpose |
| --- | --- |
| BROM | Executes from ROM in SoC; loads SPL |
| SPL | Minimal loader to init RAM, load U-Boot |
| U-Boot | Full bootloader; loads OS, DTB, initrd |